# SIMATS ENGINEERING

### ASSESSMENT TOOL 1

NAME: PAVITHRA K

REG NO: 192411027

COURSE NAME: COMPUTER NETWORKS COURSE CODE: CSA07

COURSE OUTCOME COVERED

CO5:

Design network topologies star, mesh, bus, and hybrid using networking devices, transmission media, and cabling standards

(BL2,BL5)

Assessment Tool Weightage: 40%

QUESTIONS: 3	Total Marks:30

Annexure A

## Constraint-Based Problem Solving

- A multinational company is opening a new branch office in a remote rural location where high-speed wired internet is unavailable. The office must support cloud-based applications, video conferencing, and secure communication with the headquarters. The company has a limited budget for connectivity but requires uninterrupted business operations. As a network engineer, recommend a suitable internet connection, networking devices, and security mechanisms. Justify your recommendations by considering bandwidth limitations, reliability, cost, and future scalability

- A university plans to provide campus-wide wireless internet access for more than 3,000 students across classrooms, laboratories, libraries, and hostels. The network must support a large number of simultaneous users while minimizing signal interference and ensuring secure access. Due to budget constraints, only a limited number of wireless access points can be deployed. Analyze the requirements and propose an efficient wireless network design, including access point placement, frequency band selection, and authentication methods. Explain how your solution balances coverage, performance, security, and cost.

- A financial institution is upgrading its internal computer network to improve cybersecurity while allowing employees to access sensitive customer information and online banking services. The organization must comply with strict security policies without significantly affecting network performance or increasing operational costs. Design a secure network architecture by selecting appropriate network segmentation techniques, firewall deployment, authentication methods, and intrusion detection mechanisms. Justify your design by considering the constraints of security, performance, compliance, and budget.

RUBRICS FOR CALCULATION

| Criteria | Weightage (%) | Exemplary (4) | Proficient (3) | Developing (2) | Beginning (1) |
| --- | --- | --- | --- | --- | --- |
| Understanding of Problem & Constraints | 20% | Clearly identifies all constraints and fully understands the problem requirements | Identifies most constraints with minor gaps in understanding | Identifies some constraints but misses key aspects | Shows little or no understanding of constraints or problem |
| Problem Decomposition | 15% | Breaks problem into logical, well-structured sub-problems effectively | Breaks problem into sub-parts with some logical flow | Attempts decomposition but lacks clarity or structure | No clear decomposition of the problem |
| Application of Constraints in Solution | 20% | Applies all constraints correctly and consistently in solution design | Applies most constraints correctly with minor errors | Applies some constraints but with noticeable errors | Fails to apply constraints appropriately |
| Logical Reasoning & Strategy | 15% | Uses strong, clear, and efficient reasoning strategies | Uses generally sound reasoning with minor gaps | Reasoning is partially correct but inconsistent | Reasoning is unclear or incorrect |
| Solution Accuracy & Feasibility | 15% | Solution is fully correct, feasible, and optimized | Solution is mostly correct and feasible | Solution has errors or is partially feasible | Solution is incorrect or not feasible |
| Creativity & Innovation | 5% | Demonstrates highly innovative and unique approach | Shows some creativity in approach | Limited creativity shown | No creativity; relies on basic or copied ideas |
| Clarity of Presentation | 10% | Solution is clearly presented with excellent organization and explanation | Presentation is clear with minor issues | Presentation lacks clarity or organization | Poorly presented and difficult to understand |

1. Network Design for a Remote Rural Branch Office

Introduction

A multinational company is establishing a branch office in a remote rural area where high-speed wired Internet such as fiber or cable is not available. The office needs reliable connectivity for cloud applications, video conferencing, file sharing, and secure communication with headquarters. Since the company has a limited budget but requires uninterrupted operation, a combination of 4G/5G cellular Internet as the primary connection and satellite Internet as a backup is recommended.

Proposed Internet Connection

Primary Connection – 4G/5G Cellular Internet

A business-grade 4G LTE or 5G connection can be used as the primary Internet connection if cellular coverage is available.

Advantages:

- Faster deployment than wired connections.

- No requirement for laying cables.

- Relatively low installation cost.

- 5G can provide high bandwidth and low latency where available.

- Suitable for cloud applications and video conferencing.

Backup Connection – Satellite Internet

A satellite connection can be used as a backup WAN link.

If the cellular connection fails, the router can automatically switch traffic to the satellite connection using WAN failover.

Advantages:

- Useful in remote locations.

- Independent of terrestrial cable infrastructure.

- Provides business continuity during cellular outages.

However, satellite connections may have higher latency and data costs, so they should preferably be used as a backup rather than the primary connection.

Required Networking Devices

1. Dual-WAN/SD-WAN Router

A dual-WAN router should be installed to connect both the cellular and satellite links.

Functions include:

- Load balancing

- Automatic failover

- NAT

- DHCP

- VPN support

- Traffic monitoring

- QoS

2. Managed Ethernet Switch

A managed switch connects office computers, printers, IP phones, and servers.

It also supports:

- VLANs

- Traffic management

- Port security

- QoS

3. Wireless Access Point

Business-grade Wi-Fi access points provide wireless connectivity to laptops, smartphones, and other devices.

4. Firewall

A next-generation firewall should be deployed between the Internet and internal network.

It can provide:

- Stateful packet inspection

- Application filtering

- URL filtering

- VPN

- Intrusion prevention

- Malware protection

5. UPS

A UPS should be installed for the router, firewall, switch, and access points.

It provides temporary power during power failures and protects networking equipment from voltage fluctuations.

Proposed Network Architecture

4G/5G Network

|

|

Primary WAN

|

v

+------------------+

| Dual-WAN Router  |

| / SD-WAN Router  |

+--------+---------+

|

+-----+-----+

| Firewall  |

+-----+-----+

|

Managed Switch

|

+-----------+-----------+

|           |           |

PCs       Servers       AP

|

Wi-Fi Users

Satellite Internet

|

Backup WAN

|

Dual-WAN Router

Security Mechanisms

VPN

A site-to-site IPsec VPN should be established between the branch office and headquarters.

This ensures that communication between the two offices is encrypted.

Firewall

The firewall should block unauthorized incoming traffic while allowing required business applications.

VLAN Segmentation

The internal network can be divided into VLANs:

| VLAN | Purpose |
| --- | --- |
| VLAN 10 | Employees |
| VLAN 20 | Servers |
| VLAN 30 | Guest Wi-Fi |
| VLAN 40 | Network Management |

This prevents unnecessary communication between different types of users.

Authentication

Employees should use strong passwords and preferably multi-factor authentication (MFA) for sensitive systems.

WPA3

Wireless networks should use WPA3 Enterprise where supported.

Bandwidth Management

Since rural connectivity may have limited bandwidth, Quality of Service (QoS) should prioritize important applications.

Example:

| Priority | Application |
| --- | --- |
| High | VoIP |
| High | Video conferencing |
| Medium | Cloud applications |
| Low | Software updates |
| Low | Entertainment/streaming |

This prevents non-business traffic from consuming all available bandwidth.

Reliability

Reliability can be improved through:

- 4G/5G primary connection

- Satellite backup

- Automatic WAN failover

- UPS

- Dual-WAN router

- Network monitoring

If the primary connection fails, traffic automatically moves to the backup connection.

Cost and Scalability

The design avoids expensive wired infrastructure while still providing redundancy.

Future upgrades can include:

- 5G upgrade

- Additional cellular provider

- SD-WAN

- More access points

- Higher-capacity firewall

- Cloud-managed networking

Conclusion

The recommended solution is a 4G/5G-based primary Internet connection with satellite backup, controlled through a dual-WAN/SD-WAN router. A firewall, managed switch, secure Wi-Fi, VLANs, VPN, QoS, and UPS provide security, performance, and reliability. This design offers a good balance between bandwidth, reliability, cost, and future scalability for a remote branch office.

2. Wireless Network Design for a University with 3,000+ Students

Introduction

A university needs campus-wide wireless connectivity for more than 3,000 students across classrooms, laboratories, libraries, and hostels. The main challenge is supporting a large number of simultaneous users while minimizing interference and maintaining security. Since the number of access points is limited by budget, the network must use careful AP placement, appropriate frequency bands, centralized management, and efficient authentication.

Network Requirements

The proposed network should provide:

- Campus-wide coverage

- High user capacity

- Low interference

- Secure authentication

- Good roaming

- Efficient bandwidth utilization

- Centralized management

- Scalability

Proposed Wireless Architecture

Internet

|

+--------------+

| Core Router  |

+------+-------+

|

+--------------+

| Core Switch  |

+------+-------+

|

Wireless Controller

|

+---------------+---------------+

|               |               |

AP1             AP2             AP3

|               |               |

Classrooms       Laboratories      Library

|

More APs

|

Hostels

A centralized wireless controller or cloud-managed WLAN platform can manage all access points.

Access Point Placement

Since the number of APs is limited, APs should not simply be installed based on building size. Placement should be based on:

- Number of users

- Building structure

- Wall thickness

- Expected traffic

- Interference

- Application requirements

Classrooms

APs should be positioned so that each high-density classroom area receives sufficient coverage.

Laboratories

Laboratories require higher capacity because many computers and wireless devices may operate simultaneously.

Libraries

Libraries require stable coverage and sufficient capacity because students may use laptops and smartphones simultaneously.

Hostels

Hostels should have APs distributed across floors or high-density areas rather than using a single high-power AP.

Frequency Band Selection

2.4 GHz

The 2.4 GHz band provides better coverage and penetration through walls, but it has fewer non-overlapping channels and is more susceptible to interference.

Therefore, it should be used mainly for devices that require greater range.

5 GHz

5 GHz provides:

- Higher throughput

- More channels

- Lower interference

- Better performance for high-density environments

Therefore, most modern student devices should preferably use 5 GHz.

6 GHz

If the university's equipment and regulatory environment support Wi-Fi 6E/7, the 6 GHz band can provide additional capacity and cleaner spectrum.

Channel Planning

APs should not all use the same channel.

A proper channel plan should be implemented to reduce co-channel and adjacent-channel interference.

For 2.4 GHz, commonly used non-overlapping channels are:

Channel 1

Channel 6

Channel 11

For 5 GHz and 6 GHz, additional channels can be used depending on regulatory availability and equipment support.

Automatic RF management can dynamically select appropriate channels and transmit power.

User Capacity

Because more than 3,000 students may connect simultaneously, the university should avoid designing the network only for coverage.

The design should consider capacity per AP.

For example:

Students

|

v

High-Density Areas

|

v

Multiple Strategically Placed APs

|

v

Wireless Controller

|

v

Core Network

|

v

Internet

AP transmit power should be controlled so that neighboring APs do not unnecessarily interfere with each other.

Authentication and Security

WPA3-Enterprise

WPA3-Enterprise should be preferred for student and staff networks.

It provides stronger enterprise-level wireless security.

802.1X Authentication

802.1X can authenticate users through a centralized authentication server such as RADIUS.

Students can use their university credentials instead of sharing a common Wi-Fi password.

Separate Networks

Different SSIDs/VLANs can be created:

| Network | Purpose |
| --- | --- |
| Student Wi-Fi | Students |
| Staff Wi-Fi | Faculty and staff |
| Guest Wi-Fi | Visitors |
| IoT Network | Smart devices |

VLAN segmentation prevents guest or student devices from directly accessing sensitive university systems.

Bandwidth Management

The university should implement:

- QoS

- Per-user bandwidth limits

- Application prioritization

- Traffic monitoring

For example, academic applications can receive higher priority than recreational streaming.

Cost Optimization

Because only a limited number of APs can be installed:

- Deploy APs in high-density locations first.

- Use centralized wireless management.

- Prefer dual/tri-band APs.

- Use automatic channel selection.

- Adjust transmit power.

- Reuse channels carefully.

- Avoid unnecessarily high AP density.

This provides maximum performance from a limited number of devices.

Balancing Coverage and Performance

The design balances the four major requirements as follows:

| Requirement | Solution |
| --- | --- |
| Coverage | Strategic AP placement |
| Performance | 5 GHz/6 GHz and capacity planning |
| Interference | Channel and power management |
| Security | WPA3 + 802.1X |
| Cost | Deploy APs according to user density |
| Scalability | Centralized controller and VLANs |

Conclusion

An efficient university wireless network should use strategically positioned high-capacity access points, primarily 5 GHz/6 GHz for modern devices, controlled transmit power, proper channel planning, and centralized WLAN management. WPA3-Enterprise and 802.1X provide secure authentication, while VLANs isolate students, staff, guests, and IoT devices. This design provides a practical balance between coverage, performance, security, interference reduction, and budget.

3. Secure Network Architecture for a Financial Institution

Introduction

Financial institutions handle highly sensitive information such as customer details, account information, transactions, and banking credentials. Therefore, the network must provide strong security while maintaining high performance and meeting regulatory requirements.

A suitable solution is a defense-in-depth architecture using network segmentation, next-generation firewalls, multi-factor authentication, intrusion detection/prevention, encryption, and centralized monitoring.

Proposed Network Architecture

The network should be divided into multiple security zones.

Internet

|

+---------------+

| Edge Firewall |

+-------+-------+

|

DMZ

+-----------+-----------+

|                       |

Web Servers              Email/Public

|                   Services

|

Internal Firewall

|

+-------+-------+

|               |

Employee VLANs    Server VLANs

|               |

|        +------+------+

|        |             |

Staff PCs  Database    Application

Servers      Servers

|

Restricted

Data Network

Security Monitoring / IDS / SIEM

|

SOC / Admin Team

1. Network Segmentation

Network segmentation is one of the most important security mechanisms.

The network can be divided into:

Employee VLAN

Used for normal employee computers.

Server VLAN

Contains application servers and internal services.

Database VLAN

Contains sensitive customer databases.

Access should be restricted to authorized application servers.

Management VLAN

Used only for network administrators and management devices.

Guest VLAN

Guest users should have Internet access but no access to internal financial systems.

DMZ

Public-facing servers such as web servers should be placed in a DMZ.

This prevents direct access from the Internet to the internal network.

VLAN Example

| VLAN | Purpose | Security |
| --- | --- | --- |
| VLAN 10 | Employees | Controlled access |
| VLAN 20 | Applications | Restricted |
| VLAN 30 | Database | Highly restricted |
| VLAN 40 | Management | Admin only |
| VLAN 50 | Guest | Internet only |
| VLAN 60 | Security monitoring | Security team |

2. Firewall Deployment

A next-generation firewall (NGFW) should be deployed at the Internet edge.

It should provide:

- Stateful inspection

- Application control

- Access control

- VPN

- Intrusion prevention

- Malware filtering

- Logging

A second internal firewall can be placed between sensitive server networks and general employee networks.

Example

Internet

|

Edge Firewall

|

DMZ

|

Internal Firewall

|

Internal Network

|

Database Firewall/ACL

|

Customer Database

This provides multiple security layers.

3. Authentication

Multi-Factor Authentication

MFA should be used for:

- Administrators

- Remote employees

- VPN users

- Access to sensitive applications

Authentication can combine:

Something you know → Password

+

Something you have → Security token/phone

802.1X

802.1X can be used for network access control.

Employees must authenticate before receiving access to the corporate network.

Role-Based Access Control

Users should receive permissions according to their job roles.

For example:

Employee → Customer information required for job

Manager → Additional reports

Administrator → Network management

Guest → Internet only

This follows the principle of least privilege.

4. Intrusion Detection and Prevention

An IDS/IPS should monitor network traffic for suspicious activity.

IDS

An Intrusion Detection System detects suspicious activity and generates alerts.

IPS

An Intrusion Prevention System can automatically block detected malicious traffic.

The IDS/IPS should monitor:

- Port scans

- Malware traffic

- Brute-force attempts

- Suspicious login activity

- Unauthorized connections

- Network attacks

5. Security Monitoring

Security logs from firewalls, servers, authentication systems, and IDS/IPS should be collected centrally.

A SIEM platform can correlate these events and generate security alerts.

Firewall ──────┐

IDS/IPS ───────┤

Servers ───────┤

Authentication ├──> SIEM ──> Security Team

Network Devices ┘

This allows the security team to identify attacks more quickly.

6. Encryption

Sensitive communications should be protected using encryption.

Examples include:

- TLS for web applications

- IPsec VPN for remote connections

- Encryption for sensitive data

- Secure administrative protocols such as SSH

Unencrypted protocols should be avoided wherever possible.

7. Performance Considerations

Security mechanisms should not significantly reduce network performance.

This can be achieved by:

- Using hardware-accelerated firewalls

- Applying security inspection selectively

- Using high-performance switches

- Using VLANs to reduce unnecessary broadcast traffic

- Monitoring network utilization

- Prioritizing critical banking applications with QoS

8. Compliance

The financial institution should maintain:

- Strong access control

- Audit logs

- Data protection

- Security monitoring

- Regular vulnerability assessment

- Incident response procedures

- Periodic security audits

The exact regulatory requirements depend on the country and the institution's services, but the architecture supports common financial-sector security principles such as least privilege, segmentation, encryption, monitoring, and auditability.

9. Budget Considerations

Instead of deploying expensive security appliances everywhere, security should be concentrated around the most critical areas.

For example:

Internet → NGFW

↓

DMZ

↓

Internal Firewall

↓

VLAN Segmentation

↓

IDS/IPS + SIEM

Existing managed switches can often be reused for VLAN segmentation, reducing infrastructure costs.

Open-source or appropriately sized monitoring solutions can also be considered where they meet the organization's security and compliance requirements.

10. Defense-in-Depth Approach

The proposed security architecture uses several layers:

Layer 1 → Perimeter Firewall

Layer 2 → DMZ

Layer 3 → Internal Firewall

Layer 4 → VLAN Segmentation

Layer 5 → Authentication + MFA

Layer 6 → IDS/IPS

Layer 7 → Encryption

Layer 8 → SIEM Monitoring

If one security mechanism fails, other mechanisms continue to protect the network.

Conclusion

The financial institution should implement a segmented, defense-in-depth network architecture using DMZs, VLANs, internal and external firewalls, MFA, 802.1X, IDS/IPS, encryption, and centralized SIEM monitoring. Sensitive databases should be placed in highly restricted network segments, while guest and employee traffic should be isolated. This architecture provides a strong balance between security, performance, compliance, operational cost, and scalability.