## SIMATS ENGINEERING

### ASSESSMENT TOOL 2

NAME: PAVITHRA K

REG NO: 192411027

COURSE NAME: COMPUTER NETWORKS COURSE CODE: CSA07

COURSE OUTCOME COVERED

CO5: Measure network performance using key metrics with simulation tools.

(BL4,BL5)

Assessment Tool Weightage: 30%

Annexure A

# Pseudocode Writing Task (Algorithm Design)

- In a network simulation using Cisco Packet Tracer, a student records total data received as 8000 bits in 4 seconds and observes that 200 packets were sent while 180 packets were received. Write a pseudocode algorithm to calculate the network throughput in bits per second and the packet loss percentage based on the given values. The algorithm should include validation to ensure that time and total packets sent are not zero to avoid errors. It should display both the throughput and packet loss results clearly. Add logic to classify the network as Efficient if throughput is greater than 1500 bps and packet loss is less than 10%, otherwise classify it as Inefficient. Finally, ensure the algorithm outputs a complete summary of the network performance based on the calculated metrics.

- An organization has multiple network links connecting its branches. The administrator wants to monitor bandwidth utilization and identify overloaded links. Write a pseudocode algorithm that periodically collects the bandwidth usage of each network link and stores the values in an array or list. Use a loop to continuously monitor all links. Add conditions to detect when bandwidth utilization exceeds 80% and recommend switching traffic to a backup link. Explain how this algorithm supports efficient bandwidth management and prevents network congestion.

- A company has six branch offices connected through multiple network links. Each link has different bandwidth, delay, and utilization values, which change throughout the day. The network administrator wants to continuously identify the best communication path between the headquarters and each

branch based on overall link quality rather than just the shortest distance.Write a pseudocode algorithm that periodically collects the bandwidth, delay, and utilization of every network link and stores the values in suitable arrays or lists. Use a loop to repeat the monitoring process at regular intervals. Calculate a link quality score for each path using the collected metrics and select the path with the highest score. Include conditions to detect when a link becomes overloaded or fails, automatically selecting an alternate path and generating an alert for the administrator. Explain how your algorithm improves network reliability, routing efficiency, and fault tolerance while adapting to changing network conditions.

RUBRICS FOR CALCULATION

| Criteria | Weightag e | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Identifies all inputs, outputs, constraints accurately | Minor missing elements | Partial understandin g | Incorrect interpretatio n |
| Algorithm Design | 30% | Complete, correct, and logically sound solution | Minor logical gaps | Partially correct logic | Incorrect approach |
| Use of Control Structures | 20% | Efficient and appropriate use of loops, conditions, functions/modu les | Minor inefficiencie s | Limited or basic usage | Incorrect or missing constructs |
| Pseudocode Structure | 10% | Well-structured , clear, consistent format, easy to follow | Mostly clear with minor issues | Difficult to follow in parts | Poorly structured and unclear |
| Completeness | 20% | Handles all cases; optimized approach | Handles most cases; reasonable efficiency | Handles basic cases only | Incomplete or inefficient |

1. Network Design for a Remote Rural Branch Office

Introduction

A multinational company is establishing a branch office in a remote rural area where high-speed wired Internet such as fiber or cable is not available. The office needs reliable connectivity for cloud applications, video conferencing, file sharing, and secure communication with headquarters. Since the company has a limited budget but requires uninterrupted operation, a combination of 4G/5G cellular Internet as the primary connection and satellite Internet as a backup is recommended.

Proposed Internet Connection

Primary Connection – 4G/5G Cellular Internet

A business-grade 4G LTE or 5G connection can be used as the primary Internet connection if cellular coverage is available.

Advantages:

- Faster deployment than wired connections.

- No requirement for laying cables.

- Relatively low installation cost.

- 5G can provide high bandwidth and low latency where available.

- Suitable for cloud applications and video conferencing.

Backup Connection – Satellite Internet

A satellite connection can be used as a backup WAN link.

If the cellular connection fails, the router can automatically switch traffic to the satellite connection using WAN failover.

Advantages:

- Useful in remote locations.

- Independent of terrestrial cable infrastructure.

- Provides business continuity during cellular outages.

However, satellite connections may have higher latency and data costs, so they should preferably be used as a backup rather than the primary connection.

Required Networking Devices

1. Dual-WAN/SD-WAN Router

A dual-WAN router should be installed to connect both the cellular and satellite links.

Functions include:

- Load balancing

- Automatic failover

- NAT

- DHCP

- VPN support

- Traffic monitoring

- QoS

2. Managed Ethernet Switch

A managed switch connects office computers, printers, IP phones, and servers.

It also supports:

- VLANs

- Traffic management

- Port security

- QoS

3. Wireless Access Point

Business-grade Wi-Fi access points provide wireless connectivity to laptops, smartphones, and other devices.

4. Firewall

A next-generation firewall should be deployed between the Internet and internal network.

It can provide:

- Stateful packet inspection

- Application filtering

- URL filtering

- VPN

- Intrusion prevention

- Malware protection

5. UPS

A UPS should be installed for the router, firewall, switch, and access points.

It provides temporary power during power failures and protects networking equipment from voltage fluctuations.

Proposed Network Architecture

4G/5G Network

|

|

Primary WAN

|

v

+------------------+

| Dual-WAN Router  |

| / SD-WAN Router  |

+--------+---------+

|

+-----+-----+

| Firewall  |

+-----+-----+

|

Managed Switch

|

+-----------+-----------+

|           |           |

PCs       Servers       AP

|

Wi-Fi Users

Satellite Internet

|

Backup WAN

|

Dual-WAN Router

Security Mechanisms

VPN

A site-to-site IPsec VPN should be established between the branch office and headquarters.

This ensures that communication between the two offices is encrypted.

Firewall

The firewall should block unauthorized incoming traffic while allowing required business applications.

VLAN Segmentation

The internal network can be divided into VLANs:

| VLAN | Purpose |
| --- | --- |
| VLAN 10 | Employees |
| VLAN 20 | Servers |
| VLAN 30 | Guest Wi-Fi |
| VLAN 40 | Network Management |

This prevents unnecessary communication between different types of users.

Authentication

Employees should use strong passwords and preferably multi-factor authentication (MFA) for sensitive systems.

WPA3

Wireless networks should use WPA3 Enterprise where supported.

Bandwidth Management

Since rural connectivity may have limited bandwidth, Quality of Service (QoS) should prioritize important applications.

Example:

| Priority | Application |
| --- | --- |
| High | VoIP |
| High | Video conferencing |
| Medium | Cloud applications |
| Low | Software updates |
| Low | Entertainment/streaming |

This prevents non-business traffic from consuming all available bandwidth.

Reliability

Reliability can be improved through:

- 4G/5G primary connection

- Satellite backup

- Automatic WAN failover

- UPS

- Dual-WAN router

- Network monitoring

If the primary connection fails, traffic automatically moves to the backup connection.

Cost and Scalability

The design avoids expensive wired infrastructure while still providing redundancy.

Future upgrades can include:

- 5G upgrade

- Additional cellular provider

- SD-WAN

- More access points

- Higher-capacity firewall

- Cloud-managed networking

Conclusion

The recommended solution is a 4G/5G-based primary Internet connection with satellite backup, controlled through a dual-WAN/SD-WAN router. A firewall, managed switch, secure Wi-Fi, VLANs, VPN, QoS, and UPS provide security, performance, and reliability. This design offers a good balance between bandwidth, reliability, cost, and future scalability for a remote branch office.

2. Wireless Network Design for a University with 3,000+ Students

Introduction

A university needs campus-wide wireless connectivity for more than 3,000 students across classrooms, laboratories, libraries, and hostels. The main challenge is supporting a large number of simultaneous users while minimizing interference and maintaining security. Since the number of access points is limited by budget, the network must use careful AP placement, appropriate frequency bands, centralized management, and efficient authentication.

Network Requirements

The proposed network should provide:

- Campus-wide coverage

- High user capacity

- Low interference

- Secure authentication

- Good roaming

- Efficient bandwidth utilization

- Centralized management

- Scalability

Proposed Wireless Architecture

Internet

|

+--------------+

| Core Router  |

+------+-------+

|

+--------------+

| Core Switch  |

+------+-------+

|

Wireless Controller

|

+---------------+---------------+

|               |               |

AP1             AP2             AP3

|               |               |

Classrooms       Laboratories      Library

|

More APs

|

Hostels

A centralized wireless controller or cloud-managed WLAN platform can manage all access points.

Access Point Placement

Since the number of APs is limited, APs should not simply be installed based on building size. Placement should be based on:

- Number of users

- Building structure

- Wall thickness

- Expected traffic

- Interference

- Application requirements

Classrooms

APs should be positioned so that each high-density classroom area receives sufficient coverage.

Laboratories

Laboratories require higher capacity because many computers and wireless devices may operate simultaneously.

Libraries

Libraries require stable coverage and sufficient capacity because students may use laptops and smartphones simultaneously.

Hostels

Hostels should have APs distributed across floors or high-density areas rather than using a single high-power AP.

Frequency Band Selection

2.4 GHz

The 2.4 GHz band provides better coverage and penetration through walls, but it has fewer non-overlapping channels and is more susceptible to interference.

Therefore, it should be used mainly for devices that require greater range.

5 GHz

5 GHz provides:

- Higher throughput

- More channels

- Lower interference

- Better performance for high-density environments

Therefore, most modern student devices should preferably use 5 GHz.

6 GHz

If the university's equipment and regulatory environment support Wi-Fi 6E/7, the 6 GHz band can provide additional capacity and cleaner spectrum.

Channel Planning

APs should not all use the same channel.

A proper channel plan should be implemented to reduce co-channel and adjacent-channel interference.

For 2.4 GHz, commonly used non-overlapping channels are:

Channel 1

Channel 6

Channel 11

For 5 GHz and 6 GHz, additional channels can be used depending on regulatory availability and equipment support.

Automatic RF management can dynamically select appropriate channels and transmit power.

User Capacity

Because more than 3,000 students may connect simultaneously, the university should avoid designing the network only for coverage.

The design should consider capacity per AP.

For example:

Students

|

v

High-Density Areas

|

v

Multiple Strategically Placed APs

|

v

Wireless Controller

|

v

Core Network

|

v

Internet

AP transmit power should be controlled so that neighboring APs do not unnecessarily interfere with each other.

Authentication and Security

WPA3-Enterprise

WPA3-Enterprise should be preferred for student and staff networks.

It provides stronger enterprise-level wireless security.

802.1X Authentication

802.1X can authenticate users through a centralized authentication server such as RADIUS.

Students can use their university credentials instead of sharing a common Wi-Fi password.

Separate Networks

Different SSIDs/VLANs can be created:

| Network | Purpose |
| --- | --- |
| Student Wi-Fi | Students |
| Staff Wi-Fi | Faculty and staff |
| Guest Wi-Fi | Visitors |
| IoT Network | Smart devices |

VLAN segmentation prevents guest or student devices from directly accessing sensitive university systems.

Bandwidth Management

The university should implement:

- QoS

- Per-user bandwidth limits

- Application prioritization

- Traffic monitoring

For example, academic applications can receive higher priority than recreational streaming.

Cost Optimization

Because only a limited number of APs can be installed:

- Deploy APs in high-density locations first.

- Use centralized wireless management.

- Prefer dual/tri-band APs.

- Use automatic channel selection.

- Adjust transmit power.

- Reuse channels carefully.

- Avoid unnecessarily high AP density.

This provides maximum performance from a limited number of devices.

Balancing Coverage and Performance

The design balances the four major requirements as follows:

| Requirement | Solution |
| --- | --- |
| Coverage | Strategic AP placement |
| Performance | 5 GHz/6 GHz and capacity planning |
| Interference | Channel and power management |
| Security | WPA3 + 802.1X |
| Cost | Deploy APs according to user density |
| Scalability | Centralized controller and VLANs |

Conclusion

An efficient university wireless network should use strategically positioned high-capacity access points, primarily 5 GHz/6 GHz for modern devices, controlled transmit power, proper channel planning, and centralized WLAN management. WPA3-Enterprise and 802.1X provide secure authentication, while VLANs isolate students, staff, guests, and IoT devices. This design provides a practical balance between coverage, performance, security, interference reduction, and budget.

3. Secure Network Architecture for a Financial Institution

Introduction

Financial institutions handle highly sensitive information such as customer details, account information, transactions, and banking credentials. Therefore, the network must provide strong security while maintaining high performance and meeting regulatory requirements.

A suitable solution is a defense-in-depth architecture using network segmentation, next-generation firewalls, multi-factor authentication, intrusion detection/prevention, encryption, and centralized monitoring.

Proposed Network Architecture

The network should be divided into multiple security zones.

Internet

|

+---------------+

| Edge Firewall |

+-------+-------+

|

DMZ

+-----------+-----------+

|                       |

Web Servers              Email/Public

|                   Services

|

Internal Firewall

|

+-------+-------+

|               |

Employee VLANs    Server VLANs

|               |

|        +------+------+

|        |             |

Staff PCs  Database    Application

Servers      Servers

|

Restricted

Data Network

Security Monitoring / IDS / SIEM

|

SOC / Admin Team

1. Network Segmentation

Network segmentation is one of the most important security mechanisms.

The network can be divided into:

Employee VLAN

Used for normal employee computers.

Server VLAN

Contains application servers and internal services.

Database VLAN

Contains sensitive customer databases.

Access should be restricted to authorized application servers.

Management VLAN

Used only for network administrators and management devices.

Guest VLAN

Guest users should have Internet access but no access to internal financial systems.

DMZ

Public-facing servers such as web servers should be placed in a DMZ.

This prevents direct access from the Internet to the internal network.

VLAN Example

| VLAN | Purpose | Security |
| --- | --- | --- |
| VLAN 10 | Employees | Controlled access |
| VLAN 20 | Applications | Restricted |
| VLAN 30 | Database | Highly restricted |
| VLAN 40 | Management | Admin only |
| VLAN 50 | Guest | Internet only |
| VLAN 60 | Security monitoring | Security team |

2. Firewall Deployment

A next-generation firewall (NGFW) should be deployed at the Internet edge.

It should provide:

- Stateful inspection

- Application control

- Access control

- VPN

- Intrusion prevention

- Malware filtering

- Logging

A second internal firewall can be placed between sensitive server networks and general employee networks.

Example

Internet

|

Edge Firewall

|

DMZ

|

Internal Firewall

|

Internal Network

|

Database Firewall/ACL

|

Customer Database

This provides multiple security layers.

3. Authentication

Multi-Factor Authentication

MFA should be used for:

- Administrators

- Remote employees

- VPN users

- Access to sensitive applications

Authentication can combine:

Something you know → Password

+

Something you have → Security token/phone

802.1X

802.1X can be used for network access control.

Employees must authenticate before receiving access to the corporate network.

Role-Based Access Control

Users should receive permissions according to their job roles.

For example:

Employee → Customer information required for job

Manager → Additional reports

Administrator → Network management

Guest → Internet only

This follows the principle of least privilege.

4. Intrusion Detection and Prevention

An IDS/IPS should monitor network traffic for suspicious activity.

IDS

An Intrusion Detection System detects suspicious activity and generates alerts.

IPS

An Intrusion Prevention System can automatically block detected malicious traffic.

The IDS/IPS should monitor:

- Port scans

- Malware traffic

- Brute-force attempts

- Suspicious login activity

- Unauthorized connections

- Network attacks

5. Security Monitoring

Security logs from firewalls, servers, authentication systems, and IDS/IPS should be collected centrally.

A SIEM platform can correlate these events and generate security alerts.

Firewall ──────┐

IDS/IPS ───────┤

Servers ───────┤

Authentication ├──> SIEM ──> Security Team

Network Devices ┘

This allows the security team to identify attacks more quickly.

6. Encryption

Sensitive communications should be protected using encryption.

Examples include:

- TLS for web applications

- IPsec VPN for remote connections

- Encryption for sensitive data

- Secure administrative protocols such as SSH

Unencrypted protocols should be avoided wherever possible.

7. Performance Considerations

Security mechanisms should not significantly reduce network performance.

This can be achieved by:

- Using hardware-accelerated firewalls

- Applying security inspection selectively

- Using high-performance switches

- Using VLANs to reduce unnecessary broadcast traffic

- Monitoring network utilization

- Prioritizing critical banking applications with QoS

8. Compliance

The financial institution should maintain:

- Strong access control

- Audit logs

- Data protection

- Security monitoring

- Regular vulnerability assessment

- Incident response procedures

- Periodic security audits

The exact regulatory requirements depend on the country and the institution's services, but the architecture supports common financial-sector security principles such as least privilege, segmentation, encryption, monitoring, and auditability.

9. Budget Considerations

Instead of deploying expensive security appliances everywhere, security should be concentrated around the most critical areas.

For example:

Internet → NGFW

↓

DMZ

↓

Internal Firewall

↓

VLAN Segmentation

↓

IDS/IPS + SIEM

Existing managed switches can often be reused for VLAN segmentation, reducing infrastructure costs.

Open-source or appropriately sized monitoring solutions can also be considered where they meet the organization's security and compliance requirements.

10. Defense-in-Depth Approach

The proposed security architecture uses several layers:

Layer 1 → Perimeter Firewall

Layer 2 → DMZ

Layer 3 → Internal Firewall

Layer 4 → VLAN Segmentation

Layer 5 → Authentication + MFA

Layer 6 → IDS/IPS

Layer 7 → Encryption

Layer 8 → SIEM Monitoring

If one security mechanism fails, other mechanisms continue to protect the network.

Conclusion

The financial institution should implement a segmented, defense-in-depth network architecture using DMZs, VLANs, internal and external firewalls, MFA, 802.1X, IDS/IPS, encryption, and centralized SIEM monitoring. Sensitive databases should be placed in highly restricted network segments, while guest and employee traffic should be isolated. This architecture provides a strong balance between security, performance, compliance, operational cost, and scalability.

Quick Comparison for Assessment

| Question | Main Challenge | Recommended Solution |
| --- | --- | --- |
| 1. Rural Branch | No wired high-speed Internet | 4G/5G + satellite backup + SD-WAN/VPN |
| 2. University | 3,000+ simultaneous Wi-Fi users | Strategic AP placement + 5/6 GHz + WPA3/802.1X |
| 3. Financial Institution | Highly sensitive data | VLAN/DMZ segmentation + NGFW + MFA + IDS/IPS + SIEM |

Key points to remember for the assessment

Q1:
4G/5G → Dual-WAN → Firewall → Switch → VLANs/APs → VPN → Satellite backup

Q2:
Strategic AP placement → 5/6 GHz → Channel planning → WPA3 → 802.1X → VLANs

Q3:
Internet → Firewall → DMZ → Internal Firewall → VLANs → Restricted Database → IDS/IPS → SIEM

Today 12:38 AM

You said:

Pseudocode Writing Task (Algorithm Design)

1.     In a
network simulation using Cisco Packet Tracer, a student records total data
received as 8000 bits in 4 seconds and observes that 200 packets were sent
while 180 packets were received. Write a pseudocode algorithm to calculate the
network throughput in bits per second and the packet loss percentage based on
the given values. The algorithm should include validation to ensure that time
and total packets sent are not zero to avoid errors. It should display both the
throughput and packet loss results clearly. Add logic to classify the network
as Efficient if throughput is greater than 1500 bps and packet loss is less than 10%, otherwise classify
it as Inefficient. Finally, ensure the algorithm outputs a complete
summary of the network performance based on the calculated metrics.
2.    
An organization has multiple network links connecting
its branches. The administrator wants to monitor bandwidth utilization and identify
overloaded links. Write a pseudocode algorithm that periodically
collects the bandwidth usage of each network link and stores the values in an
array or list. Use a loop to continuously monitor all links. Add conditions to
detect when bandwidth utilization exceeds 80% and recommend switching traffic
to a backup link. Explain how this algorithm supports efficient bandwidth
management and prevents network congestion.
3.     A company
has six branch offices connected
through multiple network
links. Each link has different bandwidth, delay, and utilization values,
which change throughout the day. The network administrator wants to continuously identify the best communication path between the headquarters and each

branch
based on overall link quality rather than just the shortest distance.Write a
pseudocode algorithm that periodically collects the bandwidth, delay, and
utilization of every network link and stores
the values in suitable arrays
or lists. Use a loop to repeat the monitoring process at regular intervals.
Calculate a link quality score for each path using the collected metrics and
select the path with the highest score. Include conditions to detect when a
link becomes overloaded or fails, automatically selecting an alternate path and
generating an alert for the administrator. Explain how your algorithm improves
network reliability, routing efficiency, and fault tolerance while
adapting to changing network conditions.
Pseudocode Writing Task (Algorithm Design)
1.     In a
network simulation using Cisco Packet Tracer, a student records total data
received as 8000 bits in 4 seconds and observes that 200 packets were sent
while 180 packets were received. Write a pseudocode algorithm to calculate the
network throughput in bits per second and the packet loss percentage based on
the given values. The algorithm should include validation to ensure that time
and total packets sent are not zero to avoid errors. It should display both the
throughput and packet loss results clearly. Add logic to classify the network
as Efficient if throughput is greater than 1500 bps and packet loss is less than 10%, otherwise classify
it as Inefficient. Finally, ensure the algorithm outputs a complete
summary of the network performance based on the calculated metrics.

2.    
An organization has multiple network links connecting
its branches. The administrator wants to monitor bandwidth utilization and identify
overloaded links. Write a pseudocode algorithm that periodically
collects the bandwidth usage of each network link and stores the values in an
array or list. Use a loop to continuously monitor all links. Add conditions to
detect when bandwidth utilization exceeds 80% and recommend switching traffic
to a backup link. Explain how this algorithm supports efficient bandwidth
management and prevents network congestion.

3.     A company
has six branch offices connected
through multiple network
links. Each link has different bandwidth, delay, and utilization values,
which change throughout the day. The network administrator wants to continuously identify the best communication path between the headquarters and each

branch
based on overall link quality rather than just the shortest distance.Write a
pseudocode algorithm that periodically collects the bandwidth, delay, and
utilization of every network link and stores
the values in suitable arrays
or lists. Use a loop to repeat the monitoring process at regular intervals.
Calculate a link quality score for each path using the collected metrics and
select the path with the highest score. Include conditions to detect when a
link becomes overloaded or fails, automatically selecting an alternate path and
generating an alert for the administrator. Explain how your algorithm improves
network reliability, routing efficiency, and fault tolerance while
adapting to changing network conditions.
 GIVE COMPLETE ANSWER

Show more

ChatGPT said:

Pseudocode Writing Task – Complete Answers

1. Network Throughput and Packet Loss Calculation

Problem Understanding

Given:

- Total data received = 8000 bits

- Time = 4 seconds

- Packets sent = 200

- Packets received = 180

Formula

Throughput:

Packet Loss:

The network is classified as Inefficient, because although throughput is greater than 1500 bps, packet loss is 10%, and the condition requires packet loss to be less than 10%.

Pseudocode

ALGORITHM NetworkPerformance

BEGIN

// Input values

dataReceived ← 8000

time ← 4

packetsSent ← 200

packetsReceived ← 180

// Validation

IF time = 0 THEN

DISPLAY "Error: Time cannot be zero."

STOP

END IF

IF packetsSent = 0 THEN

DISPLAY "Error: Total packets sent cannot be zero."

STOP

END IF

// Validate packet values

IF packetsReceived < 0 OR packetsReceived > packetsSent THEN

DISPLAY "Error: Invalid packet values."

STOP

END IF

// Calculate throughput

throughput ← dataReceived / time

// Calculate packet loss

lostPackets ← packetsSent - packetsReceived

packetLoss ← (lostPackets / packetsSent) × 100

// Display calculated results

DISPLAY "----- NETWORK PERFORMANCE -----"

DISPLAY "Total Data Received: ", dataReceived, " bits"

DISPLAY "Time: ", time, " seconds"

DISPLAY "Packets Sent: ", packetsSent

DISPLAY "Packets Received: ", packetsReceived

DISPLAY "Packets Lost: ", lostPackets

DISPLAY "Throughput: ", throughput, " bps"

DISPLAY "Packet Loss: ", packetLoss, "%"

// Classify network

IF throughput > 1500 AND packetLoss < 10 THEN

status ← "Efficient"

ELSE

status ← "Inefficient"

END IF

DISPLAY "Network Classification: ", status

// Complete summary

DISPLAY "----- PERFORMANCE SUMMARY -----"

IF status = "Efficient" THEN

DISPLAY "The network has good throughput and acceptable packet loss."

ELSE

DISPLAY "The network requires improvement in throughput or packet loss."

END IF

END

Output for the Given Values

----- NETWORK PERFORMANCE -----

Total Data Received: 8000 bits

Time: 4 seconds

Packets Sent: 200

Packets Received: 180

Packets Lost: 20

Throughput: 2000 bps

Packet Loss: 10%

Network Classification: Inefficient

----- PERFORMANCE SUMMARY -----

The network requires improvement in throughput or packet loss.

Explanation

The algorithm first validates the input values to avoid division-by-zero errors. It then calculates throughput using the amount of data received divided by the time taken. Packet loss is calculated from the difference between packets sent and received. Finally, the algorithm compares the calculated values with the given thresholds. Since 2000 bps > 1500 bps, but 10% is not less than 10%, the network is classified as Inefficient.

2. Bandwidth Utilization Monitoring and Overload Detection

Problem Understanding

An organization has multiple network links connecting different branches. The administrator wants to continuously monitor bandwidth utilization.

The algorithm should:

- Store network links in an array/list.

- Collect bandwidth utilization periodically.

- Store utilization values.

- Check every link.

- Detect utilization above 80%.

- Recommend switching traffic to a backup link.

- Continue monitoring continuously.

Pseudocode

ALGORITHM BandwidthMonitoring

BEGIN

// List of network links

links ← ["Link1", "Link2", "Link3", "Link4"]

// Array to store bandwidth utilization

utilization ← [0, 0, 0, 0]

// Backup links

backupLinks ← ["Backup1", "Backup2", "Backup3", "Backup4"]

WHILE TRUE DO

DISPLAY "----- BANDWIDTH MONITORING -----"

// Monitor every network link

FOR i ← 0 TO LENGTH(links) - 1 DO

// Collect current bandwidth utilization

utilization[i] ← GET_BANDWIDTH_UTILIZATION(links[i])

DISPLAY links[i],

" Utilization = ", utilization[i], "%"

// Check for overload

IF utilization[i] > 80 THEN

DISPLAY "WARNING: ", links[i],

" is overloaded."

DISPLAY "Utilization = ",

utilization[i], "%"

DISPLAY "Recommendation: Switch traffic to ",

backupLinks[i]

SWITCH_TRAFFIC(

links[i],

backupLinks[i]

)

DISPLAY "Traffic redirected to backup link."

ELSE

DISPLAY links[i],

" is operating normally."

END IF

END FOR

DISPLAY "Monitoring completed."

// Wait before next monitoring cycle

WAIT 10 seconds

END WHILE

END

Example Output

----- BANDWIDTH MONITORING -----

Link1 Utilization = 65%

Link1 is operating normally.

Link2 Utilization = 84%

WARNING: Link2 is overloaded.

Recommendation: Switch traffic to Backup2

Traffic redirected to backup link.

Link3 Utilization = 72%

Link3 is operating normally.

Link4 Utilization = 91%

WARNING: Link4 is overloaded.

Recommendation: Switch traffic to Backup4

Traffic redirected to backup link.

Monitoring completed.

Explanation

The algorithm continuously checks the bandwidth utilization of all available links. The utilization values are stored in an array, making it easy to compare and analyze the links.

When utilization is 80% or less, the link is considered to be operating normally.

When utilization becomes greater than 80%, the algorithm identifies the link as overloaded and recommends moving traffic to a backup link.

How It Supports Efficient Bandwidth Management

1. Continuous monitoring:
The network is monitored at regular intervals rather than only checking it manually.

2. Early overload detection:
The 80% threshold provides an early warning before the link becomes completely congested.

3. Traffic redirection:
Traffic can be moved to a backup link when required.

4. Reduced congestion:
Overloaded links are relieved before performance becomes severely degraded.

5. Better resource utilization:
Available backup capacity can be used more effectively.

6. Scalability:
More links can easily be added to the array.

Advantages

Continuous Monitoring

↓

Collect Utilization

↓

Store in Array

↓

Check Each Link

↓

Utilization > 80%?

/       \

YES        NO

↓          ↓

Switch to     Continue

Backup        Monitoring

↓

Reduce Congestion

Conclusion

This algorithm provides a simple and effective method for bandwidth management and congestion prevention. By continuously monitoring utilization and switching traffic when a link exceeds 80%, network performance and availability can be improved.

3. Dynamic Best-Path Selection Based on Link Quality

Problem Understanding

The company has six branch offices connected to headquarters through multiple network links.

Each link has:

- Bandwidth

- Delay

- Utilization

- Status

These values change throughout the day.

Therefore, selecting a path only according to distance is not sufficient. The algorithm should calculate a link quality score based on multiple network parameters and select the path with the highest score.

Proposed Link Quality Formula

A simple normalized scoring formula can be used:

Where:

Therefore:

- Higher bandwidth → better score

- Lower delay → better score

- Lower utilization → better score

The weights can be changed depending on the organization's requirements.

Pseudocode

ALGORITHM DynamicBestPathSelection

BEGIN

// Six branch offices

branches ← [

"Branch1",

"Branch2",

"Branch3",

"Branch4",

"Branch5",

"Branch6"

]

// Network links

links ← [

"Link1",

"Link2",

"Link3",

"Link4",

"Link5",

"Link6",

"Link7",

"Link8"

]

// Arrays to store network metrics

bandwidth ← []

delay ← []

utilization ← []

status ← []

qualityScore ← []

// Monitoring interval

monitoringInterval ← 10 seconds

WHILE TRUE DO

DISPLAY "================================"

DISPLAY "DYNAMIC NETWORK MONITORING"

DISPLAY "================================"

// Step 1: Collect link information

FOR each link IN links DO

bandwidth[link] ← GET_BANDWIDTH(link)

delay[link] ← GET_DELAY(link)

utilization[link] ← GET_UTILIZATION(link)

status[link] ← GET_STATUS(link)

END FOR

// Step 2: Calculate maximum values

maxBandwidth ← MAX(bandwidth)

maxDelay ← MAX(delay)

// Step 3: Calculate quality score

FOR each link IN links DO

IF status[link] = "FAILED" THEN

qualityScore[link] ← 0

ELSE

bandwidthScore ←

(bandwidth[link] / maxBandwidth) × 100

delayScore ←

(1 - (delay[link] / maxDelay)) × 100

utilizationScore ←

(1 - (utilization[link] / 100)) × 100

qualityScore[link] ←

(0.5 × bandwidthScore) +

(0.3 × delayScore) +

(0.2 × utilizationScore)

END IF

END FOR

// Step 4: Detect overloaded or failed links

FOR each link IN links DO

IF status[link] = "FAILED" THEN

DISPLAY "ALERT: ", link,

" has failed."

GENERATE_ALERT(link, "LINK FAILURE")

ELSE IF utilization[link] > 80 THEN

DISPLAY "WARNING: ", link,

" is overloaded."

GENERATE_ALERT(link, "HIGH UTILIZATION")

END IF

END FOR

// Step 5: Find the best path for every branch

FOR each branch IN branches DO

availablePaths ←

FIND_PATHS("Headquarters", branch)

bestPath ← NULL

highestScore ← -1

FOR each path IN availablePaths DO

pathScore ← 0

pathAvailable ← TRUE

// Calculate total path score

FOR each link IN path DO

IF status[link] = "FAILED" THEN

pathAvailable ← FALSE

ELSE IF utilization[link] > 80 THEN

pathAvailable ← FALSE

ELSE

pathScore ←

pathScore + qualityScore[link]

END IF

END FOR

// Select highest scoring available path

IF pathAvailable = TRUE AND

pathScore > highestScore THEN

highestScore ← pathScore

bestPath ← path

END IF

END FOR

// Step 6: Apply selected path

IF bestPath ≠ NULL THEN

SET_ROUTE(

"Headquarters",

branch,

bestPath

)

DISPLAY "Branch: ", branch

DISPLAY "Best Path: ", bestPath

DISPLAY "Path Score: ", highestScore

ELSE

DISPLAY "ALERT: No available path for ",

branch

GENERATE_ALERT(

branch,

"NO AVAILABLE PATH"

)

END IF

END FOR

// Step 7: Wait and monitor again

DISPLAY "Waiting for next monitoring cycle..."

WAIT monitoringInterval

END WHILE

END

Example of Link Quality Calculation

Suppose three possible links have the following values:

| Link | Bandwidth | Delay | Utilization | Status |
| --- | --- | --- | --- | --- |
| Link A | 100 Mbps | 20 ms | 40% | Active |
| Link B | 80 Mbps | 10 ms | 30% | Active |
| Link C | 50 Mbps | 40 ms | 90% | Active |

Although Link A has the highest bandwidth, Link B has lower delay and utilization.

The algorithm considers all three parameters instead of simply selecting the link with the largest bandwidth or shortest distance.

Link C is automatically treated as problematic because its utilization is 90%, which exceeds the 80% threshold.

Dynamic Path Selection Process

START

|

↓

Collect Link Metrics

|

+-------+-------+

|       |       |

Bandwidth  Delay  Utilization

|       |       |

+-------+-------+

|

↓

Calculate Score

|

↓

Check Link Status

/        \

Failed      Active

|            |

Score=0     Check Load

|

Utilization > 80%?

/          \

YES           NO

|             |

Generate Alert   Available

|             |

+------┬------+

↓

Find All Paths

|

↓

Compare Path Scores

|

↓

Select Best Path

|

↓

Update Routing

|

↓

Wait 10 Seconds

|

↓

Monitor Again

Handling Link Failure

Suppose the current path is:

Headquarters → Link1 → Link3 → Branch4

If Link3 fails, the algorithm detects:

Link3 Status = FAILED

It then:

- Generates an alert.

- Removes Link3 from available paths.

- Searches for alternate paths.

- Calculates the quality score of each alternate path.

- Selects the highest-scoring available path.

- Updates the routing information.

Example:

Original Path

HQ → Link1 → Link3 → Branch4

X

FAILED

Alternate Path

HQ → Link2 → Link5 → Branch4

↓

Selected

This allows communication to continue without waiting for manual intervention.

Handling Overloaded Links

If:

Utilization > 80%

the algorithm generates a warning.

For example:

Link5 Utilization = 87%

WARNING!

Link5 is overloaded.

Searching for alternate path...

Alternate Path Found:

HQ → Link2 → Link6 → Branch5

Traffic redirected.

This prevents heavily utilized links from becoming congestion points.

How the Algorithm Improves the Network

1. Network Reliability

The algorithm continuously checks link health. If a link fails, an alternate path is automatically selected.

2. Routing Efficiency

Instead of selecting a path only based on physical distance, the algorithm considers:

- Bandwidth

- Delay

- Utilization

- Link availability

Therefore, it can choose a path that provides better overall performance.

3. Fault Tolerance

Multiple paths are maintained between headquarters and branches. If one path fails, another available path can be selected.

4. Congestion Prevention

Links exceeding 80% utilization are detected early. Traffic can be redirected to less-loaded links.

5. Dynamic Adaptation

Network conditions can change throughout the day. Because the algorithm periodically collects fresh values, the selected path can change according to current conditions.

6. Automatic Alerts

The administrator receives alerts when:

- A link fails.

- A link becomes overloaded.

- No valid path exists.

7. Scalability

The same algorithm can be extended to more than six branches and a larger number of network links by adding them to the arrays/lists.