# SIMATS ENGINEERING

## ASSESSMENT TOOL 3

### COURSE NAME: Computer Networks	COURSE CODE:	CSA0716

### REG NO: 192411027                                                       NAME: PAVITHRA K

### COURSE OUTCOME COVERED

CO5: Measure network performance using key metrics with simulation tools

Assessment Tool Weightage: 20%

### QUESTIONS: 40	Total Marks:40

Annexure A Mock Interview

### QUESTIONS:40	Total Marks:40

- Analyze how network congestion affects packet delay and packet loss. (BL4)(1)

- Compare the impact of star and mesh topologies on network reliability and fault tolerance. (BL4)(1)

- Why does increasing the number of routers between source and destination increase end-to-end delay? Explain logically. (BL4)(1)

- Analyze how Quality of Service (QoS) improves the performance of voice and video traffic. (BL4)(1)

- Evaluate the impact of using fiber optic cables instead of copper cables for high-speed communication. (BL5)(1)

- Analyze how network segmentation using VLANs improves performance and security. (BL4)(1)

- Compare the effects of TCP and UDP on network performance for different applications. (BL4)(1)

- Why is load balancing important in large enterprise networks? Analyze its impact on performance and availability. (BL5)(1)

- Analyze how frequent routing updates influence network efficiency and bandwidth utilization. (BL4)(1)

- Evaluate the trade-off between network security measures (such as firewalls and encryption) and overall network performance. (BL5)(1)

- A network has high delay—how will you identify and fix the issue? (BL4)(1)

- If throughput is low despite high bandwidth, what could be the reasons? (BL4)(1)

- How would you reduce packet loss in a congested network? (BL4)(1)

- A VoIP call is breaking frequently—how would you troubleshoot? (BL4)(1)

- Given a network with high jitter, suggest improvements (BL4)(1)

- How will you simulate a congested network in Packet Tracer? (BL5)(1)

- If packets are delayed in a router, what steps will you take? (BL4)(1)

- How would you optimize a network for online gaming? (BL4)(1)

- Design a small network and explain how you will measure its performance (BL5)(1)

- If RTT is very high, how will you diagnose the issue? (BL4)(1)

- Explain network performance to a non-technical person (BL5)(1)

- Describe throughput using a real-life example (BL5)(1)

- Present how packet loss affects video streaming (BL5)(1)

- Explain delay types clearly in under 1 minute (BL4)(1)

- Describe Cisco Packet Tracer in a professional way (BL5)(1)

- Explain QoS in simple and technical terms (BL5)(1)

- How would you present your simulation results to a client? (BL4)(1)

- Explain jitter using a real-world analogy (BL5)(1)

- Describe how you tested network performance in a lab (BL4)(1)

- Explain the importance of performance metrics in one structured answer (BL5)(1)

- Introduce yourself and explain your knowledge of network performance (BL5)(1)

- Why are you interested in networking and simulation tools? (BL5)(1)

- What makes you confident in analyzing network performance? (BL5)(1)

- Explain a concept you are very strong in (related to networking) (BL5)(1)

- How would you handle a question you don’t know? (BL5)(1)

- Describe a situation where you solved a technical problem (BL4)(1)

- Why should we select you for a networking role? (BL5)(1)

- What challenges did you face while learning Packet Tracer? (BL5)(1)

- How do you improve your technical skills regularly? (BL5)(1)

- Demonstrate confidence while explaining a complex concept simply (BL5)(1)

### RUBRICS FOR CALCULATION

| Criteria | Weightag e (%) | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Conceptual Understandin g | 30% | Demonstrates thorough understandin g; answers all questions accurately | Shows good understandin g with few errors | Partial understandin g; several incorrect responses | Lacks understandin g; majority of answers incorrect |
| Accuracy of Answers | 25% | All answers are correct | Most answers are correct with minor mistakes | Some correct answers; frequent errors | Mostly incorrect answers |
| Application of Knowledge | 20% | Correctly applies concepts to problem-base d questions | Applies concepts with minor errors | Limited ability to apply concepts | Unable to apply concepts |
| Time Management | 15% | Completes quiz well within time efficiently | Completes on time with minor delays | Requires extra time or rushes through questions | Unable to complete within given time |
| Question Interpretation | 10% | Interprets all questions correctly and responds appropriately | Misinterprets a few questions | Often misinterprets questions | Unable to understand questions |

1. Analyze how network congestion affects packet delay and packet loss.

Network congestion occurs when network traffic exceeds the available capacity. Packets wait in router queues, increasing delay and jitter. If the queues become full, packets are dropped, causing packet loss and retransmissions.

2. Compare star and mesh topologies on network reliability and fault tolerance.

In a star topology, failure of the central switch can affect the entire network, although failure of one individual link affects only one device. A mesh topology has multiple paths between devices, so it provides much higher reliability and fault tolerance. However, mesh is more expensive and complex.

3. Why does increasing the number of routers between source and destination increase end-to-end delay?

Every router introduces processing, queuing, and transmission delay. Therefore, as the number of routers increases, these delays accumulate, increasing the total end-to-end delay.

4. Analyze how QoS improves the performance of voice and video traffic.

QoS prioritizes delay-sensitive traffic such as voice and video over less important traffic. It reduces delay, jitter, and packet loss, resulting in clearer voice calls and smoother video.

5. Evaluate the impact of using fiber optic cables instead of copper cables.

Fiber provides much higher bandwidth, longer transmission distances, lower signal attenuation, and immunity to electromagnetic interference. Its main disadvantages are higher installation cost and more specialized equipment. For high-speed backbone communication, fiber is generally the better choice.

6. Analyze how network segmentation using VLANs improves performance and security.

VLANs divide a physical network into logical networks. They reduce unnecessary broadcast traffic, improving performance, while also isolating departments or users and preventing unauthorized access between network segments.

7. Compare TCP and UDP on network performance for different applications.

TCP provides reliable, ordered delivery using acknowledgements and retransmissions, but this adds overhead and latency. UDP is faster and has lower overhead but does not guarantee delivery. TCP is suitable for web and file transfers, while UDP is useful for VoIP, streaming, and gaming.

8. Why is load balancing important in large enterprise networks?

Load balancing distributes traffic across multiple links or servers instead of overloading one resource. It improves performance, availability, scalability, and fault tolerance, and allows traffic to continue if one resource fails.

9. Analyze how frequent routing updates influence network efficiency and bandwidth utilization.

Routing updates help routers maintain accurate network information and respond to topology changes. However, excessively frequent updates consume bandwidth and router processing resources. Therefore, routing protocols should provide timely updates without creating unnecessary control traffic.

10. Evaluate the trade-off between security measures and network performance.

Firewalls, encryption, and intrusion detection improve security but require additional processing and can introduce some latency. The solution is to use appropriately sized security devices and optimized security policies so that strong protection is achieved without significantly reducing performance.

11. A network has high delay—how will you identify and fix the issue?

I would first use ping and traceroute to identify where the delay occurs. Then I would check bandwidth utilization, router CPU, interface errors, congestion, and routing paths. Finally, I would optimize routing, increase bandwidth, reduce congestion, or replace faulty equipment.

12. If throughput is low despite high bandwidth, what could be the reasons?

Possible reasons include packet loss, congestion, high latency, inefficient protocols, hardware limitations, interference, or server limitations. High theoretical bandwidth does not guarantee high actual throughput.

13. How would you reduce packet loss in a congested network?

I would identify the congested links, apply QoS, prioritize important traffic, use load balancing, increase bandwidth if required, and remove unnecessary traffic. I would also check for faulty cables, interfaces, and devices.

14. A VoIP call is breaking frequently—how would you troubleshoot?

I would check packet loss, latency, jitter, bandwidth, and signal quality. I would use ping and network monitoring tools, then apply QoS to prioritize VoIP traffic and check whether congestion or faulty network equipment is causing the problem.

15. Given a network with high jitter, suggest improvements.

I would reduce congestion, prioritize real-time traffic using QoS, use stable network paths, increase bandwidth where necessary, and check for wireless interference. The objective is to maintain consistent packet delivery timing.

16. How will you simulate a congested network in Packet Tracer?

I would create multiple communicating devices and generate significant traffic through a common link. I can then reduce the available link bandwidth and observe packet delay, queueing, packet loss, and throughput. I would compare the results before and after congestion.

17. If packets are delayed in a router, what steps will you take?

First, I would check router CPU and memory utilization. Then I would examine interface errors, queue length, bandwidth utilization, routing configuration, and packet statistics. I would resolve congestion or configuration problems and verify the improvement using ping and other measurements.

18. How would you optimize a network for online gaming?

I would prioritize gaming traffic using QoS, reduce latency and jitter, avoid congested routes, use reliable wired connections where possible, and ensure sufficient upload and download bandwidth. I would also minimize unnecessary background traffic.

19. Design a small network and explain how you will measure its performance.

I would connect several PCs to a switch, connect the switch to a router, and connect the router to another network.

PC1 ─┐

PC2 ─┼── Switch ── Router ── Server

PC3 ─┘

I would measure throughput, delay, packet loss, jitter, bandwidth utilization, and RTT using tools such as ping, traceroute, and Packet Tracer simulation statistics.

20. If RTT is very high, how will you diagnose the issue?

I would use ping to measure RTT and traceroute to identify the hop where delay increases. Then I would check congestion, routing, link utilization, router processing, and physical link problems. After fixing the issue, I would repeat the measurements.

21. Explain network performance to a non-technical person.

Network performance is basically how quickly and reliably information travels from one place to another. A good network is like a smooth highway: vehicles move quickly with few interruptions. A congested network is like a traffic jam, where everything becomes slower.

22. Describe throughput using a real-life example.

Throughput is the actual amount of data successfully transferred per second. For example, if a water pipe delivers 100 liters of water every second, the delivery rate is similar to network throughput. A network may have high capacity, but actual throughput can be lower because of congestion or packet loss.

23. Present how packet loss affects video streaming.

Packet loss means some video data does not reach the destination. This can cause buffering, freezing, pixelation, or reduced video quality. High packet loss makes the streaming experience unstable.

24. Explain delay types clearly in under 1 minute.

There are four major types of network delay. Processing delay is the time a device takes to examine a packet. Queuing delay occurs when a packet waits in a queue. Transmission delay is the time required to place the packet onto the link. Propagation delay is the time required for the signal to travel through the physical medium.

25. Describe Cisco Packet Tracer in a professional way.

Cisco Packet Tracer is a network simulation and learning platform that allows users to design, configure, and troubleshoot virtual networks. It supports devices such as routers and switches and helps students understand networking concepts without requiring physical hardware.

26. Explain QoS in simple and technical terms.

In simple terms, QoS is a system that gives priority to important network traffic. Technically, Quality of Service classifies and manages traffic using mechanisms such as prioritization, queuing, shaping, and policing to control delay, jitter, packet loss, and bandwidth usage.

27. How would you present your simulation results to a client?

I would present the topology, objectives, test conditions, performance metrics, results, and recommendations. I would use simple tables or graphs to compare expected and actual performance and explain the business impact clearly.

28. Explain jitter using a real-world analogy.

Imagine a bus that should arrive every 10 minutes. If it arrives after 10 minutes, then 20 minutes, then 5 minutes, the arrival time is inconsistent. This variation is similar to network jitter, where packets arrive at different time intervals.

29. Describe how you tested network performance in a lab.

I created a network topology in Packet Tracer, configured IP addresses and routing, generated traffic, and measured performance. I evaluated throughput, delay, packet loss, RTT, and utilization under normal and congested conditions.

30. Explain the importance of performance metrics in one structured answer.

Network performance metrics help us understand whether a network is operating efficiently. Throughput measures successful data transfer, delay measures delivery time, packet loss measures dropped packets, and jitter measures variation in packet arrival time. Together, these metrics help identify problems and improve network quality.

31. Introduce yourself and explain your knowledge of network performance.

Sample answer:

“Good morning, I am Pavithra, a Computer Science and Engineering student. I have a good understanding of computer networking concepts such as IP addressing, routing, TCP/IP, VLANs, QoS, throughput, delay, packet loss, and jitter. I have also used Cisco Packet Tracer to design and analyze network simulations. I am interested in applying these concepts to solve practical networking problems.”

32. Why are you interested in networking and simulation tools?

“I am interested in networking because it explains how different systems communicate and exchange information. Simulation tools such as Cisco Packet Tracer allow me to design networks, test configurations, identify problems, and understand network behavior without requiring physical hardware.”

33. What makes you confident in analyzing network performance?

“I understand the major performance metrics such as throughput, delay, packet loss, jitter, and RTT. I follow a systematic approach: first identify the metric showing a problem, then locate its source, apply a solution, and finally compare the results before and after the improvement.”

34. Explain a concept you are very strong in related to networking.

“I am particularly comfortable with network performance analysis. I understand how throughput, delay, packet loss, jitter, and bandwidth are related. For example, when congestion increases, packets wait in queues, which increases delay and can eventually cause packet loss.”

35. How would you handle a question you don't know?

“I would be honest and say that I am not completely sure about the answer. I would explain what I know about the related concept and, if necessary, say that I would verify the information using reliable technical documentation. I believe it is better to be accurate than to give an incorrect answer confidently.”

36. Describe a situation where you solved a technical problem.

“While working with a network simulation, I faced a connectivity problem where devices could not communicate correctly. I checked the IP addresses, subnet masks, interfaces, and routing configuration step by step. After identifying the configuration issue, I corrected it and verified connectivity using ping. This taught me the importance of systematic troubleshooting.”

37. Why should we select you for a networking role?

“You should select me because I have a strong foundation in networking concepts and I am willing to learn continuously. I can analyze network problems systematically, use simulation tools such as Packet Tracer, and understand performance metrics. I am also a responsible learner who is willing to improve my practical skills.”

38. What challenges did you face while learning Packet Tracer?

“Initially, I found device configuration, IP addressing, routing, and connecting the correct interfaces challenging. I overcame these difficulties by practicing different topologies and troubleshooting connectivity step by step. This improved my confidence in configuring and analyzing networks.”

39. How do you improve your technical skills regularly?

“I improve my technical skills through regular practice, laboratory exercises, coding, network simulations, and studying technical documentation. I try to understand the concept first and then implement it practically. I also learn from errors because troubleshooting helps strengthen my understanding.”

40. Demonstrate confidence while explaining a complex concept simply.

Example: Explaining congestion confidently

“Network congestion is similar to traffic congestion on a road. If too many vehicles enter a road with limited capacity, they slow down and some may not reach their destination efficiently. Similarly, when too much data enters a network, packets wait in router queues, increasing delay and jitter. If the queues become full, packets are dropped, causing packet loss. We can reduce congestion using QoS, load balancing, traffic management, or additional bandwidth.”