## SIMATS ENGINEERING

### ASSESSMENT TOOL 2

NAME: PAVITHRA K

REG NO: 192411027

COURSE NAME: COMPUTER NETWORKS      COURSE CODE: CSA0716

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 30%

Annexure A

# Scenario-Based

- A newly established Smart Campus is deploying a secure network infrastructure for its academic and administrative departments. The network administrator has been allocated the IP address block 192.168.100.0/24. Based on the expected number of devices, the following subnet masks have already been assigned:

School of Computing → /25 School of Electronics → /26

School of Mechanical Engineering → /27

As the network engineer, design the subnet allocation by performing the following tasks:

  - Determine	the	network	address	and	broadcast	address	for	each department.

  - Identify the valid host IP address range for each subnet.

  - Calculate the number of usable host addresses available in each subnet.

  - Determine the remaining unused IP address range within the given network block.

- A multinational software company is expanding its headquarters and has been assigned the private IP block 10.10.0.0/16. Different departments require different subnet sizes based on the number of employees. The network administrator has allocated the following subnet masks:

Software Development → /24 Quality Assurance → /25 Human Resources → /26 Executive Management → /27

As the network administrator, perform the following tasks:

  - Determine	the	network	address	and broadcast address for each department.

  - Identify the valid range of host IP addresses.

  - Calculate the number of usable hosts in each subnet.

  - Identify the unused IP address range remaining within the allocated IP block.

- A smart hospital is migrating its network infrastructure from IPv4 to IPv6 to support thousands of connected medical devices and IoT sensors. The hospital has been assigned the IPv6 network: 2001:0db8:abcd:0000:0000:0000:0000:0000/64

As the network engineer responsible for the migration, perform the following:

  - Convert the given IPv6 address into its compressed notation.

  - Expand	the	compressed	address	back	into	its	full	hexadecimal representation.

  - Identify the network prefix using the /64 prefix length.

  - Calculate the total number of host addresses supported within the subnet.

  - the total number of possible host addresses available within this network.

- An Internet Service Provider (ISP) connects multiple customer networks through a core router. The router maintains the following routing table entries: 192.168.1.0/24

192.168.2.0/24

192.168.3.0/24

A packet arrives at the router with the destination IP address:

192.168.2.45

As the routing engineer, determine:

  - The destination subnet to which the packet belongs.

### The matching routing table entry.

  - The next-hop network/interface used to forward the packet.

  - The default route that should be used if the destination does not match any routing table entry.

  - A company establishes two office departments connected through a Layer-3 router. The allocated subnet information is as follows:

Administration LAN → 192.168.10.0/26

Finance LAN → 192.168.10.64/26

The router interfaces are configured with:

Administration Gateway → 192.168.10.1

Finance Gateway → 192.168.10.65

As the network administrator, perform the following tasks:

    - Determine the network address and broadcast address for each LAN.

    - Identify the valid host IP address range.

    - Suggest suitable IP addresses for three computers in each LAN.

    - Identify the default gateway that hosts in each LAN should use for communication.

RUBRICS FOR CALCULATION

| Scenario Based Assessment |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Criteria | Weightage (%) | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning ) |
| Understanding of Scenario | 20% | Demonstrates clear and complete understandin g of the scenario and context | Shows good understandi ng with minor gaps | Partial understandin g; misses key aspects | Misinterpret s or fails to understand the scenario |
| Identification of Key Issues | 20% | Accurately identifies all relevant issues and factors involved | Identifies most key issues with minor omissions | Identifies limited issues; some irrelevant points | Unable to identify key issues |
| Application of Concepts | 25% | Applies appropriate concepts effectively to address the scenario | Applies concepts with minor errors | Limited or partially correct application | Unable to apply relevant concepts |
| Decision Making | 20% | Makes well-reasoned , logical decisions with strong rationale | Makes reasonable decisions with some justification | Makes weak decisions with limited justification | No clear decisions made or rationale provided |
| Clarity of Response | 15% | Response is clear, structured, and logically presented | Generally clear with minor issues | Somewhat unclear or poorly structured | Disorganize d and difficult to understand |

1. Smart Campus Network (192.168.100.0/24)

Given:

- Network: 192.168.100.0/24

- School of Computing → /25

- School of Electronics → /26

- School of Mechanical Engineering → /27

Step 1: School of Computing (/25)

Subnet Mask: 255.255.255.128

- Network Address: 192.168.100.0

- Broadcast Address: 192.168.100.127

- Host Range: 192.168.100.1 – 192.168.100.126

- Usable Hosts: 27-2=126

Step 2: School of Electronics (/26)

Next subnet starts at 192.168.100.128

Subnet Mask: 255.255.255.192

- Network Address: 192.168.100.128

- Broadcast Address: 192.168.100.191

- Host Range: 192.168.100.129 – 192.168.100.190

- Usable Hosts: 26-2=62

Step 3: School of Mechanical Engineering (/27)

Next subnet starts at 192.168.100.192

Subnet Mask: 255.255.255.224

- Network Address: 192.168.100.192

- Broadcast Address: 192.168.100.223

- Host Range: 192.168.100.193 – 192.168.100.222

- Usable Hosts: 25-2=30

Step 4: Remaining Unused Range

Unused IP addresses:

192.168.100.224 – 192.168.100.255

Final Table

| Department | Subnet | Network Address | Host Range | Broadcast | Usable Hosts |
| --- | --- | --- | --- | --- | --- |
| Computing | /25 | 192.168.100.0 | 192.168.100.1 – 192.168.100.126 | 192.168.100.127 | 126 |
| Electronics | /26 | 192.168.100.128 | 192.168.100.129 – 192.168.100.190 | 192.168.100.191 | 62 |
| Mechanical | /27 | 192.168.100.192 | 192.168.100.193 – 192.168.100.222 | 192.168.100.223 | 30 |

Unused Range: 192.168.100.224 – 192.168.100.255

2. Software Company (10.10.0.0/16)

Given:

- Network: 10.10.0.0/16

- Software Development → /24

- Quality Assurance → /25

- Human Resources → /26

- Executive Management → /27

Software Development (/24)

- Network: 10.10.0.0

- Broadcast: 10.10.0.255

- Host Range: 10.10.0.1 – 10.10.0.254

- Usable Hosts: 254

Quality Assurance (/25)

Starts at 10.10.1.0

- Network: 10.10.1.0

- Broadcast: 10.10.1.127

- Host Range: 10.10.1.1 – 10.10.1.126

- Usable Hosts: 126

Human Resources (/26)

Starts at 10.10.1.128

- Network: 10.10.1.128

- Broadcast: 10.10.1.191

- Host Range: 10.10.1.129 – 10.10.1.190

- Usable Hosts: 62

Executive Management (/27)

Starts at 10.10.1.192

- Network: 10.10.1.192

- Broadcast: 10.10.1.223

- Host Range: 10.10.1.193 – 10.10.1.222

- Usable Hosts: 30

Remaining Unused Range

10.10.1.224 – 10.10.255.255

Final Table

| Department | Subnet | Network | Host Range | Broadcast | Usable Hosts |
| --- | --- | --- | --- | --- | --- |
| Software Development | /24 | 10.10.0.0 | 10.10.0.1 – 10.10.0.254 | 10.10.0.255 | 254 |
| Quality Assurance | /25 | 10.10.1.0 | 10.10.1.1 – 10.10.1.126 | 10.10.1.127 | 126 |
| Human Resources | /26 | 10.10.1.128 | 10.10.1.129 – 10.10.1.190 | 10.10.1.191 | 62 |
| Executive Management | /27 | 10.10.1.192 | 10.10.1.193 – 10.10.1.222 | 10.10.1.223 | 30 |

Unused Range: 10.10.1.224 – 10.10.255.255

3. Smart Hospital IPv6

Given:

2001:0db8:abcd:0000:0000:0000:0000:0000/64

Step 1: Compressed Address

2001:db8:abcd::/64

Step 2: Expanded Address

2001:0db8:abcd:0000:0000:0000:0000:0000

Step 3: Network Prefix

2001:db8:abcd:0::/64

Step 4: Host Bits

128 − 64 = 64 bits

Step 5: Total Host Addresses

264

= 18,446,744,073,709,551,616

Final Answer

| Item | Value |
| --- | --- |
| Compressed Address | 2001:db8:abcd::/64 |
| Expanded Address | 2001:0db8:abcd:0000:0000:0000:0000:0000 |
| Network Prefix | 2001:db8:abcd:0::/64 |
| Host Bits | 64 |
| Total Host Addresses | 18,446,744,073,709,551,616 |

4. ISP Routing

Routing Table

- 192.168.1.0/24

- 192.168.2.0/24

- 192.168.3.0/24

Destination IP

192.168.2.45

Step 1: Destination Subnet

Belongs to

192.168.2.0/24

Step 2: Matching Route

192.168.2.0/24

Step 3: Next Hop

The router forwards the packet using the next-hop/interface configured for the 192.168.2.0/24 route.

Step 4: Default Route

If no route matches,

Router uses:

0.0.0.0/0 (Default Route)

Final Answer

| Item | Answer |
| --- | --- |
| Destination Subnet | 192.168.2.0/24 |
| Matching Route | 192.168.2.0/24 |
| Next Hop | Interface/Next hop configured for 192.168.2.0/24 |
| Default Route | 0.0.0.0/0 |

5. Company LAN Configuration

Given

Administration LAN

192.168.10.0/26

Finance LAN

192.168.10.64/26

Gateways

Administration → 192.168.10.1

Finance → 192.168.10.65

Administration LAN

- Network Address: 192.168.10.0

- Broadcast Address: 192.168.10.63

- Host Range: 192.168.10.1 – 192.168.10.62

Example Computer IPs

- PC1 → 192.168.10.2

- PC2 → 192.168.10.3

- PC3 → 192.168.10.4

Gateway

192.168.10.1

Finance LAN

- Network Address: 192.168.10.64

- Broadcast Address: 192.168.10.127

- Host Range: 192.168.10.65 – 192.168.10.126

Example Computer IPs

- PC1 → 192.168.10.66

- PC2 → 192.168.10.67

- PC3 → 192.168.10.68

Gateway

192.168.10.65