# SIMATS ENGINEERING

## ASSESSMENT TOOL 3

NAME: PAVITHRA K

REG NO: 192411027

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA0716

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 30% Software:Cisco Packet Tracer,Wireshark

Annexure A

## Debugging & Optimisation Task

- A network administrator has configured a workstation with the following settings:

  - IP Address: 192.168.10.65

  - Subnet Mask: 255.255.255.192 (/26)

  - Default Gateway: 192.168.10.1

However, the workstation is unable to communicate with other systems configured in the subnet 192.168.10.0/26.

Task

Debug the network configuration by:

  - Identifying the subnet to which the configured IP address belongs.

  - Determining the network address, broadcast address, and valid host range.

  - Verifying whether the configured IP address and gateway are valid.

  - Identifying the configuration errors.

  - Optimizing the configuration by assigning an appropriate IP address and default gateway.

  - Calculating the total number of usable host addresses after correction.

- Two devices are configured with the following IPv6 addresses:

  - Device A: 2001:db8::ff00:42:8329/64

  - Device B: 2001:db8:0:0:1::1/64

Although both devices are connected to the same network, communication fails.

Task

Debug the IPv6 configuration by:

  - Expanding both IPv6 addresses into their complete notation.

  - Determining the network prefix of each device.

  - Verifying whether both devices belong to the same subnet.

  - Identifying the addressing or prefix mismatch.

  - Optimizing the IPv6 configuration by assigning appropriate addresses.

  - Calculating the number of available host addresses within the corrected subnet.

- An organization has allocated the network 192.168.1.0/24 into several departments using the subnet masks /26, /27, and /28. Some departments report insufficient IP addresses, while others have a large number of unused addresses.

Task

Analyze and optimize the subnet allocation by:

  - Identifying the network address, broadcast address, and host range for each subnet.

  - Calculating the number of usable and unused IP addresses.

  - Identifying inefficient subnet allocations.

  - Debugging the addressing plan by locating subnet allocation mistakes.

  - Recommending an optimized subnetting scheme using VLSM to improve address utilization while supporting current departmental requirements.

- Two devices are configured with the following IPv6 addresses:

  - Device A: 2001:db8::ff00:42:8329/64

  - Device B: 2001:db8:0:0:1::1/64

Although both devices are connected to the same network, communication fails. Task

Debug the IPv6 configuration by:

  - Expanding both IPv6 addresses into their complete notation.

  - Determining the network prefix of each device.

  - Verifying whether both devices belong to the same subnet.

  - Identifying the addressing or prefix mismatch.

  - Optimizing the IPv6 configuration by assigning appropriate addresses.

  - Calculating the number of available host addresses within the corrected subnet.

- A router contains the following routing table.

| Destination Network | Next Hop |
| --- | --- |
| 192.168.1.0/24 | 10.0.0.2 |
| 192.168.2.0/24 | 10.0.0.3 |
| Default Route | 10.0.0.1 |

Users report that packets destined for 192.168.2.100 are not reaching the intended network. Task

Debug the routing configuration by:

  - Identifying the matching routing entry using the longest-prefix match.

  - Determining the next-hop router selected for packet forwarding.

  - Verifying whether the destination IP belongs to the intended subnet.

  - Identifying any routing configuration errors.

  - Optimizing the routing table to improve packet forwarding efficiency.

  - Identifying the route that will be used if no matching destination exists.

### RUBRICS FOR CALCULATION

| Criteria | Weightage | Level 4 (Exemplary) | Level 3 (Proficient ) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Problem Identification | 20% | Accurately identifies all errors and root causes with clear understandin g | Identifies most errors with minor gaps | Identifies some errors but misses key issues | Unable to identify errors correctly |

| Debugging | 25% | Applies systematic debugging techniques effectively to resolve issues | Uses appropriate debugging methods with minor errors | Limited debugging approach; requires guidance | Unable to debug or resolve issues |
| --- | --- | --- | --- | --- | --- |
| Optimization | 20% | Optimizes code by significantly improving time complexity using efficient algorithms | Improves time complexity with minor gaps in efficiency | Limited improvement in time complexity | No improvement; inefficient approach retained |
| Analysis | 20% | Demonstrates strong logical reasoning and clear justification of solutions | Shows reasonable analysis with some justification | Limited analysis; weak reasoning | No analytical reasoning demonstrated |
| Code Quality | 15% | Produces clean, well-structure d code with proper comments and documentatio n | Code is mostly clear with minor documentat ion gaps | Code lacks structure and proper documentatio n | Poorly written code with no documentatio n |

1. Debugging IPv4 Network Configuration

Given

- IP Address: 192.168.10.65

- Subnet Mask: 255.255.255.192 (/26)

- Default Gateway: 192.168.10.1

Step 1: Identify the subnet

For /26, block size = 64.

Subnets are:

- 192.168.10.0 – 192.168.10.63

- 192.168.10.64 – 192.168.10.127

- 192.168.10.128 – 192.168.10.191

- 192.168.10.192 – 192.168.10.255

IP 192.168.10.65 belongs to:

192.168.10.64/26

Step 2: Network Details

- Network Address: 192.168.10.64

- Broadcast Address: 192.168.10.127

- Valid Host Range: 192.168.10.65 – 192.168.10.126

Step 3: Verify IP Address

Configured IP:

192.168.10.65

✔ Valid host address.

Step 4: Verify Gateway

Gateway:

192.168.10.1

This belongs to subnet

192.168.10.0/26

Workstation belongs to

192.168.10.64/26

❌ Gateway is in a different subnet.

Step 5: Configuration Error

- Incorrect default gateway.

- Workstation cannot communicate because the gateway is outside its subnet.

Step 6: Optimized Configuration

| Setting | Correct Value |
| --- | --- |
| IP Address | 192.168.10.65 |
| Subnet Mask | 255.255.255.192 |
| Default Gateway | 192.168.10.126 (or any valid router IP in 192.168.10.64/26) |

Step 7: Usable Hosts

232-26-2=26-2=64-2=62

Usable Hosts = 62

Final Answer

- Subnet: 192.168.10.64/26

- Network Address: 192.168.10.64

- Broadcast Address: 192.168.10.127

- Host Range: 192.168.10.65 – 192.168.10.126

- Error: Wrong default gateway

- Correct Gateway: 192.168.10.126

- Usable Hosts: 62

2. Debugging IPv6 Configuration

Given

Device A

2001:db8::ff00:42:8329/64

Device B

2001:db8:0:0:1::1/64

Step 1: Expand Addresses

Device A

2001:0db8:0000:0000:0000:ff00:0042:8329

Device B

2001:0db8:0000:0000:0001:0000:0000:0001

Step 2: Network Prefix

Device A

Network Prefix

2001:db8:0:0::/64

Device B

Network Prefix

2001:db8:0:0::/64

Step 3: Same Subnet?

Yes.

Both have

2001:db8:0:0::/64

✔ Same subnet.

Step 4: Problem

No addressing mismatch.

Communication failure is likely due to:

- IPv6 disabled

- Firewall blocking traffic

- Duplicate address

- Interface down

- Incorrect router configuration

Step 5: Optimized Configuration

Example

Device A

2001:db8:0:0::10/64

Device B

2001:db8:0:0::20/64

Gateway

2001:db8:0:0::1

Step 6: Host Addresses

Host bits = 64

264

= 18,446,744,073,709,551,616

Final Answer

- Expanded addresses shown above

- Network Prefix: 2001:db8:0:0::/64

- Same subnet: Yes

- Issue: Not an addressing mismatch; likely another network configuration issue

- Correct example addresses: 2001:db8:0:0::10/64 and 2001:db8:0:0::20/64

- Available Hosts: 2⁶⁴

3. Optimizing Subnet Allocation using VLSM

Given

Network

192.168.1.0/24

Subnets

- /26

- /27

- /28

Current Allocation

/26

Network

192.168.1.0

Broadcast

192.168.1.63

Host Range

192.168.1.1 – 192.168.1.62

Usable Hosts

62

/27

Network

192.168.1.64

Broadcast

192.168.1.95

Host Range

192.168.1.65 – 192.168.1.94

Usable Hosts

30

/28

Network

192.168.1.96

Broadcast

192.168.1.111

Host Range

192.168.1.97 – 192.168.1.110

Usable Hosts

14

Unused Addresses

Remaining

192.168.1.112 – 192.168.1.255

Problem

- Some departments have more IPs than needed.

- Others have too few.

- Address wastage.

Optimized VLSM

Allocate according to requirement:

| Department | Needed Hosts | Recommended Subnet |
| --- | --- | --- |
| Largest | 50 | /26 |
| Medium | 25 | /27 |
| Small | 10 | /28 |

This minimizes unused addresses while meeting current needs.

Final Answer

- /26 → 62 hosts

- /27 → 30 hosts

- /28 → 14 hosts

- Unused addresses should be reassigned using VLSM for better utilization.

4. Debugging IPv6 Configuration

This question is identical to Question 2.

Answer

- Device A (Expanded): 2001:0db8:0000:0000:0000:ff00:0042:8329

- Device B (Expanded): 2001:0db8:0000:0000:0001:0000:0000:0001

- Network Prefix (both): 2001:db8:0:0::/64

- Same subnet: Yes

- No prefix mismatch.

- Suggested addresses:

  - Device A: 2001:db8:0:0::10/64

  - Device B: 2001:db8:0:0::20/64

- Available host addresses: 2⁶⁴ = 18,446,744,073,709,551,616

5. Debugging Routing Table

Routing Table

| Destination Network | Next Hop |
| --- | --- |
| 192.168.1.0/24 | 10.0.0.2 |
| 192.168.2.0/24 | 10.0.0.3 |
| Default Route | 10.0.0.1 |

Destination IP

192.168.2.100

Step 1: Longest Prefix Match

Destination

192.168.2.100

Matches

192.168.2.0/24

Step 2: Next Hop

Selected next hop

10.0.0.3

Step 3: Verify Destination

Subnet

192.168.2.0/24

Host Range

192.168.2.1 – 192.168.2.254

Destination

192.168.2.100

✔ Valid.

Step 4: Possible Routing Errors

If packets are not reaching:

- Wrong next-hop IP

- Next-hop router is down

- Interface is down

- Missing return route

- Incorrect static route

Step 5: Optimization

- Verify next-hop 10.0.0.3 is reachable.

- Enable dynamic routing (OSPF/RIP/EIGRP) if the network is large.

- Remove invalid or duplicate routes.

- Keep routing table updated.

Step 6: Default Route

If no destination matches,

Router uses:

Default Route → 10.0.0.1