# SIMATS ENGINEERING

ASSESSMENT TOOL 1

NAME: PAVITHRA K

REG NO:192411027

COURSE NAME: COMPUTER NETWORKS COURSE CODE: CSA0716

COURSE OUTCOME COVERED

CO4:

Design network topologies star, mesh, bus, and hybrid using networking devices, transmission media, and cabling standards

(BL2,BL6)

Assessment Tool Weightage: 40%

Annexure A

Coding challenge

- Develop a Python program that generates a Star Topology for a given number of hosts connected to a central switch. The program should accept the number of hosts as input and display the topology either in ASCII format or a graphical representation. For each host, print the host label, the name of the central switch, and the corresponding Cat6 UTP cable length connecting the host to the switch. Verify that all hosts are connected through the central switch and explain how the star topology facilitates reliable communication in a local area network.

- Write a Python function named validate_segment(length_m) to verify whether a given UTP cable segment satisfies the IEEE

802.3 Ethernet standard, which specifies a maximum cable length of 100 meters. The function should return True for valid cable lengths and return False along with an appropriate error message for invalid lengths exceeding the permissible limit. Demonstrate the correctness of the program by executing it with at least three different test cases and interpret the results.

- Develop a Python program to simulate the operation of the Carrier Sense Multiple Access with Collision Detection (CSMA/CD) protocol in a star topology consisting of four hosts connected through a central switch. The program should simulate frame transmission attempts, detect transmission collisions, calculate random back-off intervals, and continue transmission after the waiting period. Record every collision event, transmission attempt, and back-off interval in a log file named csma_log.txt, and analyze how CSMA/CD minimizes data transmission conflicts in Ethernet networks.

- Develop a client-server application using Python's socket library to simulate communication between two hosts connected through a central switch in a star topology. The server should receive frames from the client, forward them appropriately, and maintain a simulated MAC Address Table. After each successful frame transmission, display the updated MAC address table showing the learned MAC addresses and associated ports. Explain how MAC learning and frame forwarding are performed in Ethernet switches.

- Design and implement a Python class named StarSwitch that models the basic functionality of an Ethernet switch. The class should include methods such as add_port(host_id) to connect a host, remove_port(host_id) to disconnect a host, and broadcast(frame) to transmit a frame to all connected ports except the source port. The program should maintain a record of active ports and log every broadcast operation, indicating the source host and the destination hosts that successfully receive the frame. Demonstrate the implementation using multiple hosts and explain the significance of broadcasting in Ethernet communication.

RUBRICS FOR CALCULATION

| Criteria | Weightage | Exemplary (4) | Proficient (3) | Developing (2) | Beginning (1) |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 15% | Fully understands the problem and requirements; no clarification needed | Understands most requirements with minor clarification | Partial understanding; misses some requirements | Poor understanding of the problem |
| Algorithm Design | 20% | Efficient, optimal, and well-structured algorithm | Correct algorithm with minor inefficiencies | Basic approach but not optimal | Incorrect or no clear algorithm |
| Code Quality | 15% | Clean, well-organized, readable, and properly formatted code | Mostly clean code with minor issues | Code is somewhat unclear or inconsistent | Poorly written, unreadable code |
| Functionality | 20% | Code works perfectly for all test cases | Works for most test cases | Works for limited cases | Does not run or fails major cases |
| Error Handling | 10% | Handles all edge cases and errors effectively | Handles some edge cases | Limited error handling | No error handling |
| Efficiency | 10% | Highly optimized in time and space complexity | Reasonably efficient | Some inefficiencies | Highly inefficient |
| Documentation & Comments | 5% | Clear and meaningful comments and documentation | Adequate comments | Few or unclear comments | No comments |
| Testing & Debugging | 5% | Thoroughly tested with multiple cases; no bugs | Tested with some cases | Minimal testing | No testing done |

1. Star Topology Generation

Python Program

Python

Run

n = int(input("Enter number of hosts: "))

switch = "Switch"

print("\nSTAR TOPOLOGY")

print("Central Device:", switch)

for i in range(1, n + 1):

length = int(input(f"Enter cable length for Host{i} (meters): "))

print(f"Host{i} ---- {length}m Cat6 UTP ---- {switch}")

print("\nAll hosts are connected through the central switch.")

Sample Output

Enter number of hosts: 4

STAR TOPOLOGY

Central Device: Switch

Enter cable length for Host1 (meters): 10

Host1 ---- 10m Cat6 UTP ---- Switch

Enter cable length for Host2 (meters): 15

Host2 ---- 15m Cat6 UTP ---- Switch

Enter cable length for Host3 (meters): 20

Host3 ---- 20m Cat6 UTP ---- Switch

Enter cable length for Host4 (meters): 12

Host4 ---- 12m Cat6 UTP ---- Switch

All hosts are connected through the central switch.

Explanation

In a star topology, every host has a separate connection to a central switch.

- The switch manages communication between hosts.

- Failure of one cable affects only that host.

- Adding or removing a host is easy.

- Troubleshooting is simple.

- It provides reliable communication in a LAN.

2. UTP Cable Length Validation

Python Program

Python

Run

def validate_segment(length_m):

if 0 <= length_m <= 100:

return True

else:

return False, "Error: Cable length exceeds 100 meters."

tests = [50, 100, 120]

for length in tests:

result = validate_segment(length)

print(f"{length} meters: {result}")

Sample Output

50 meters: True

100 meters: True

120 meters: (False, 'Error: Cable length exceeds 100 meters.')

Explanation

The maximum permissible Ethernet copper cable segment length is 100 meters.

| Cable Length | Result |
| --- | --- |
| 50 m | Valid |
| 100 m | Valid |
| 120 m | Invalid |

The function returns True when the length is within the limit. If it exceeds 100 m, it returns False with an error message.

3. CSMA/CD Simulation

Note: Traditional CSMA/CD applies to shared, half-duplex Ethernet. Modern switched full-duplex Ethernet normally does not use CSMA/CD. This program is therefore a simulation for learning purposes.

Python Program

Python

Run

import random

import time

hosts = ["Host1", "Host2", "Host3", "Host4"]

with open("csma_log.txt", "w") as log:

for attempt in range(1, 11):

h1, h2 = random.sample(hosts, 2)

log.write(f"Transmission attempt {attempt}: {h1} and {h2}\n")

print(f"Attempt {attempt}: {h1} and {h2} trying to transmit")

if random.choice([True, False]):

print("Collision detected!")

log.write("Collision detected\n")

backoff = random.randint(1, 5)

print(f"Back-off time: {backoff} seconds")

log.write(f"Back-off interval: {backoff} seconds\n")

time.sleep(backoff)

print("Transmission resumed\n")

log.write("Transmission resumed\n")

else:

print("Transmission successful\n")

log.write("Transmission successful\n")

Sample csma_log.txt

Transmission attempt 1: Host1 and Host3

Collision detected

Back-off interval: 2 seconds

Transmission resumed

Transmission attempt 2: Host2 and Host4

Transmission successful

Transmission attempt 3: Host1 and Host2

Collision detected

Back-off interval: 4 seconds

Transmission resumed

Explanation

CSMA/CD works using these steps:

- Host checks whether the medium is free.

- Host starts transmission.

- If two hosts transmit at the same time, a collision occurs.

- The collision is detected.

- Each host waits for a random back-off interval.

- After waiting, transmission is attempted again.

Random back-off reduces the chance that the same hosts will collide repeatedly.

4. Client-Server Application with MAC Address Table

This example uses Python's socket library. The server acts as a simulated switch, while the client represents a host.

Server Program — server.py

Python

Run

import socket

server = socket.socket()

server.bind(("localhost", 5000))

server.listen(5)

mac_table = {}

print("Switch Server Started...")

print("Waiting for hosts...")

while True:

conn, addr = server.accept()

data = conn.recv(1024).decode()

if not data:

conn.close()

continue

mac, message = data.split("|", 1)

port = str(addr[1])

mac_table[mac] = port

print("\nFrame Received")

print("MAC Address:", mac)

print("Message:", message)

print("\nMAC Address Table")

print("-------------------------")

for m, p in mac_table.items():

print(m, "-> Port", p)

conn.send("Frame received and forwarded successfully".encode())

conn.close()

Client Program — client.py

Python

Run

import socket

client = socket.socket()

client.connect(("localhost", 5000))

mac = input("Enter MAC address: ")

message = input("Enter message: ")

data = mac + "|" + message

client.send(data.encode())

response = client.recv(1024).decode()

print(response)

client.close()

Sample Output

Server:

Switch Server Started...

Waiting for hosts...

Frame Received

MAC Address: AA:BB:CC:DD:01

Message: Hello Host2

MAC Address Table

-------------------------

AA:BB:CC:DD:01 -> Port 53241

Client:

Enter MAC address: AA:BB:CC:DD:01

Enter message: Hello Host2

Frame received and forwarded successfully

Explanation

An Ethernet switch uses MAC learning to build its MAC address table.

When a frame arrives:

- The switch reads the source MAC address.

- It records the source MAC and incoming port.

- It checks the destination MAC.

- If the destination is known, the frame is forwarded to the corresponding port.

- If the destination is unknown, the switch can flood the frame to appropriate ports.

Example:

MAC Address             Port

AA:BB:CC:DD:01          1

AA:BB:CC:DD:02          2

AA:BB:CC:DD:03          3

This makes communication efficient because the switch does not need to send every frame to every host once it has learned the destination location.

5. StarSwitch Class

Python Program

Python

Run

class StarSwitch:

def __init__(self):

self.active_ports = []

def add_port(self, host_id):

if host_id not in self.active_ports:

self.active_ports.append(host_id)

print(host_id, "connected")

def remove_port(self, host_id):

if host_id in self.active_ports:

self.active_ports.remove(host_id)

print(host_id, "disconnected")

def broadcast(self, frame):

source = frame["source"]

message = frame["message"]

receivers = [

host for host in self.active_ports

if host != source

]

print("\nBroadcast Operation")

print("Source:", source)

print("Frame:", message)

print("Received by:", receivers)

switch = StarSwitch()

switch.add_port("Host1")

switch.add_port("Host2")

switch.add_port("Host3")

switch.add_port("Host4")

frame = {

"source": "Host1",

"message": "Hello everyone"

}

switch.broadcast(frame)

switch.remove_port("Host3")

frame2 = {

"source": "Host2",

"message": "Network message"

}

switch.broadcast(frame2)

Sample Output

Host1 connected

Host2 connected

Host3 connected

Host4 connected

Broadcast Operation

Source: Host1

Frame: Hello everyone

Received by: ['Host2', 'Host3', 'Host4']

Host3 disconnected

Broadcast Operation

Source: Host2

Frame: Network message

Received by: ['Host1', 'Host4']

Explanation

The StarSwitch class represents a basic Ethernet switch.

- add_port() connects a host.

- remove_port() disconnects a host.

- broadcast() sends a frame to all active hosts except the source.

- active_ports stores the currently connected hosts.

Significance of Broadcasting

Broadcasting is useful when a host needs to communicate with all hosts in the local network, or when the destination MAC address is not yet known.

For example:

Host2

|

|

Host1 ---- Switch ---- Host3

|

|

Host4

If Host1 broadcasts a frame, Host2, Host3, and Host4 receive it, while Host1 does not receive its own transmitted frame.