# SIMATS ENGINEERING

ASSESSMENT TOOL 2

NAME: PAVITHRA K

REG NO: 192411027

COURSE NAME: COMPUTER NETWORKS COURSE CODE: CSA07

COURSE OUTCOME COVERED

CO4:

Design network topologies star, mesh, bus, and hybrid using networking devices, transmission media, and cabling standards

(BL2,BL6)

Assessment Tool Weightage: 30%

Annexure A

## Industry Problem-Solving Task

- A star topology is ideal for a 20-person startup because all devices connect to a central switch, making the network easy to manage and troubleshoot. At the core sits a Layer 2/Layer 3 managed switch (such as a Cisco Catalyst or Netgear ProSafe) with at least 24 ports to accommodate all workstations, printers, and VoIP phones with room to grow. Each device connects to the switch using Cat6 UTP cables, which support Gigabit speeds up to 100 meters — more than sufficient for any typical office floor plan. The switch uplinks to a router/firewall (such as a Cisco RV series or pfSense appliance) that handles internet access, DHCP, and network address translation. A Wi-Fi access point connected to the switch extends wireless coverage for laptops and mobile

devices. The key advantage of this design is fault isolation: if one workstation's cable fails, only that single node goes offline, leaving all other 19 employees unaffected. Centralised cable management also simplifies future moves, adds, and changes.

- A bank demands near-zero downtime, making a full or partial mesh topology the right choice. In a full mesh, every core network device — routers, core switches, and servers — connects directly to every other device, so no single link or node failure can bring down the network. For a bank, a partial mesh is typically deployed at the core layer (connecting all core switches and firewalls to each other) while the distribution and access layers use redundant uplinks to at least two core switches. Dual enterprise-grade routers (such as Cisco ASR or Juniper MX series) provide BGP-based redundant internet connectivity. All links between core devices use fiber optic cables — either single-mode for long distances between buildings or multi-mode OM4 within the data center — because fiber is immune to electromagnetic interference, supports very high bandwidth, and is essential for maintaining low-latency financial transactions. Redundant power supplies, UPS systems, and RAID storage arrays further complement the mesh design. The justification is straightforward: when a link fails in a mesh, traffic automatically reroutes through alternative paths in milliseconds using protocols like OSPF or BGP, satisfying regulatory requirements for uptime and data integrity.

- For a small classroom lab with a tight budget, bus topology offers the lowest hardware cost because all computers share a single common communication line — the bus — eliminating the need for a central switch or hub. A single coaxial cable (RG-58, 50-ohm) or, in a modern variation, a single Cat5e/Cat6 cable daisy-chained through an inexpensive unmanaged hub runs along the length of the room. Each computer connects to the backbone using a T-connector and a BNC connector in a classic coaxial setup, with terminators (50-ohm) placed at both ends of the cable to absorb signals and prevent reflection. For a more contemporary low-cost implementation, a single inexpensive 8-or 16-port unmanaged switch replaces the terminators and uses Cat5e UTP patch cables to each workstation — achieving bus-like simplicity at minimal cost. The primary limitation is that the network is susceptible to collisions and a single cable break disrupts all connected machines, but for a supervised classroom environment with light usage (internet browsing, file sharing), this trade-off is entirely acceptable. No dedicated server is required; a teacher's PC can share resources using built-in Windows/Linux file sharing.

- A large university campus serves thousands of users across multiple buildings — lecture halls, labs, libraries, dormitories, and administration — making a single topology inadequate. A hybrid topology combines the best elements of multiple designs: a fiber-based ring or mesh at the campus backbone level for

redundancy, a star topology within each building for manageability, and bus or tree structures within individual labs for cost-efficiency. At the campus core, high-speed fiber optic cables (single-mode, 10 Gbps or 40 Gbps) interconnect core routers and multilayer switches using a ring or partial mesh arrangement, ensuring that no single link failure isolates an entire building. Each building has its own distribution switch connected to the campus core via dual fiber uplinks. Within each building, a traditional star topology branches from the distribution switch to floor-level access switches, which then connect individual devices via Cat6 cables. This hierarchical three-layer model — core, distribution, and access — is the industry standard for enterprise and campus networks because adding a new building simply means installing a new distribution switch and running fiber to the core, with no disruption to existing infrastructure. Wireless access points throughout the campus connect via PoE (Power over Ethernet) to access switches, eliminating the need for separate power cabling. The hybrid design thus delivers redundancy at the backbone, simplicity at the building level, and scalability campus-wide.

- Designing a network for a company with multiple departments involves creating a structured and scalable architecture using switches and routers to ensure efficient communication and security. Each department (such as HR, Finance, Sales, and IT) is connected through dedicated Layer 2 switches that form the

access layer, allowing devices like computers and printers to communicate within the same department. These switches are then connected to a central Layer 3 switch or router, which acts as the core layer and enables communication between different departments through inter-VLAN routing. Virtual LANs (VLANs) are configured to logically separate each department’s network, improving security and reducing broadcast traffic. A router is also used to connect the internal network to external networks such as the internet, often with firewall functionality for security. Redundant links and backup paths can be included to ensure reliability, while proper IP addressing and subnetting are applied for efficient network management. This hierarchical design improves performance, enhances security, and allows easy expansion as the company grows.

RUBRICS FOR CALCULATION

| Criteria | Weightage | Exemplary (4) | Proficient (3) | Developing (2) | Beginning (1) |
| --- | --- | --- | --- | --- | --- |
| Problem Analysis | 15% | Clearly identifies and deeply analyzes the industry problem with all factors | Good understanding with minor gaps | Basic understanding; some key aspects missing | Poor or incorrect understanding of the problem |
| Solution Design | 20% | Innovative, practical, and industry-relevan t solution | Effective solution with minor limitations | Partially suitable solution | Inappropriate or unclear solution |
| Technical Implementation | 20% | Fully functional, accurate, and well-executed implementation | Mostly functional with minor errors | Partially implemented solution | Incomplete or non-functional implementation |
| Use of Tools & Technologies | 10% | Appropriate and advanced use of relevant tools and technologies | Correct use of tools | Limited or basic use of tools | Incorrect or no use of tools |
| Problem-Solvin g Approach | 10% | Logical, systematic, and well-structured approach | Mostly logical approach | Somewhat disorganized approach | No clear approach |
| Innovation & Creativity | 10% | Highly creative and original solution | Some creativity | Limited originality | No creativity |
| Testing & Validation | 5% | Thorough testing with real-world scenarios | Adequate testing | Minimal testing | No testing |
| Documentation & Presentation | 5% | Clear, professional, and well-documente d | Good documentation | Basic documentation | Poor or no documentation |
| Teamwork / Collaboration | 5% | Excellent coordination and | Good collaboration | Limited participation | No collaboration or contribution |

| Criteria | Weightage | Exemplary (4) | Proficient (3) | Developing (2) | Beginning (1) |
| --- | --- | --- | --- | --- | --- |
|  |  | contribution (if team-based) |  |  |  |

1. Network for a 20-Person Startup — Star Topology

Answer:

A star topology is suitable for a 20-person startup because all computers and devices are connected to a central switch. A 24-port managed switch can be used to connect computers, printers, VoIP phones, and other devices.

Components:

- 24-port Layer 2/Layer 3 managed switch

- Cat6 UTP cables

- Router/firewall for Internet access

- Wi-Fi access point

- DHCP and NAT

Advantages:

- Easy to manage and troubleshoot

- Easy to add new devices

- Failure of one cable affects only one device

- Provides good performance and reliability

- Cat6 supports Gigabit Ethernet up to 100 meters

Conclusion:
Star topology is simple, reliable, scalable, and suitable for a small office network.

2. Bank Network — Mesh Topology

Answer:

A full or partial mesh topology is suitable for a bank because banks require high availability and minimum downtime. In a mesh network, multiple devices have redundant connections.

Components:

- Core routers and switches

- Redundant firewalls

- Dual Internet routers

- Fiber-optic cables

- UPS and backup power

- RAID storage

Protocols such as OSPF and BGP can provide alternative routes when a link fails.

Advantages:

- High reliability

- Multiple backup paths

- Fault tolerance

- Fast recovery from link failures

- Suitable for critical financial transactions

Conclusion:
A partial mesh is generally preferred because it provides redundancy and reliability without the extremely high cost of a full mesh.

3. Small Classroom Lab — Bus Topology

Answer:

For a small classroom with a limited budget, a bus topology can be considered because all computers share a common communication medium.

A traditional bus network uses:

- RG-58 coaxial cable

- BNC connectors

- T-connectors

- 50-ohm terminators

Advantages:

- Low hardware cost

- Simple design

- Requires less cabling

- Suitable for small networks with light traffic

Disadvantages:

- Cable failure can affect the entire network

- More collisions can occur

- Difficult to troubleshoot

- Limited scalability

Conclusion:
Bus topology can be used for a small, low-cost classroom setup, although modern networks generally prefer switched star topology.

4. University Campus — Hybrid Topology

Answer:

A large university requires a hybrid topology because it contains many buildings and thousands of users. Different topologies can be combined according to the requirements of each area.

Structure:

Campus Core:

- Fiber-optic connections

- Ring or partial mesh

- Core routers and Layer 3 switches

Buildings:

- Distribution switches

- Dual fiber uplinks

Individual Floors:

- Star topology

- Access switches

- Cat6 cables

Wireless:

- Wi-Fi access points

- PoE connections

Advantages:

- Highly scalable

- Reliable and fault tolerant

- Easy to manage

- Supports multiple buildings

- Easy to add new buildings

- Provides high-speed communication

Conclusion:
Hybrid topology provides redundancy at the core, simplicity within buildings, and scalability across the entire campus.

5. Company with Multiple Departments — VLAN-Based Hierarchical Network

Answer:

A company with departments such as HR, Finance, Sales, and IT should use a hierarchical network with switches, routers, and VLANs.

Network Structure:

Internet

|

Router/Firewall

|

Layer 3 Switch

_________|_________

|         |         |

HR       Finance     Sales

VLAN       VLAN       VLAN

|           |          |

Switch      Switch     Switch

|           |          |

Computers   Computers  Computers

Each department can be assigned a separate VLAN.

Example:

| Department | VLAN |
| --- | --- |
| HR | VLAN 10 |
| Finance | VLAN 20 |
| Sales | VLAN 30 |
| IT | VLAN 40 |

The Layer 3 switch performs inter-VLAN routing, allowing communication between departments when required.

Advantages:

- Improves network security

- Reduces broadcast traffic

- Easy network management

- Provides better performance

- Allows department-wise separation

- Easy to expand

Conclusion:
A VLAN-based hierarchical network provides security, scalability, reliability, and efficient communication for companies with multiple departments.