SIMATS ENGINEERING

ASSESSMENT TOOL 1

# COURSE NAME:CSA0716                       COURSE CODE: Computer Networks

# COURSE OUTCOME COVERED

CO1: Apply the functions of OSI layer in enabling reliable data transmission across networks. (BL2, BL3)

Assessment Tool Weightage: 40%

Annexure A

SHORT ANSWER TEST

Code of Conduct:

I PAVITHRA K-192411027 (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

PAVITHRA K -192411027

Annexure A

# Short Answer Test

- A company needs to transfer a 150 MB design file from a client computer to a remote server over a network with a bandwidth of 50 Mbps. The Transport Layer divides the file into 1500-byte segments, while the Data Link Layer encapsulates each segment into frames by adding 40 bytes of header and trailer. Calculate the total number of segments and frames transmitted, the total transmission overhead introduced by the Data Link Layer, and estimate the total transmission time (ignoring propagation delay). Explain how the Transport and Data Link layers ensure reliable communication.

- A Transport Layer protocol uses a sliding window size of 6 frames. Each frame carries 1024 bytes of user data. If the sender transmits 48 frames, calculate the number of transmission windows required. If one frame in every window is lost and must be retransmitted, determine the total number of retransmissions. Explain how flow control improves communication efficiency.

- A 12,000-byte IP packet is transmitted through a router where the Maximum Transmission Unit (MTU) is 1500 bytes. Assuming each fragment requires a 20-byte IP header, calculate the total number of fragments created and the total header overhead introduced. Explain how the Network Layer ensures successful delivery of fragmented packets to the destination.

- A Transport Layer protocol uses a sliding window size of 6 frames. Each frame carries 1024 bytes of user data. If the sender transmits 48 frames, calculate the number of transmission windows required. If one frame in every window is lost and must be retransmitted, determine the total number of retransmissions. Explain how flow control improves communication efficiency.

- A multimedia application transfers a 600 MB video file. The Session Layer inserts a synchronization checkpoint after every 60 MB of transmitted data. During transmission, a failure occurs after 390 MB has been transferred. Determine the number of checkpoints created before failure and calculate the amount of data that must be retransmitted after recovery. Explain the importance of synchronization in the Session Layer.

- Before transmission, the Presentation Layer compresses a 900 MB file by 40%. The compressed data is encrypted, adding 5% overhead to the compressed size. Calculate the compressed file size, the encrypted file size, and the total percentage reduction compared to the original file. Explain how the Presentation Layer improves transmission efficiency and security.

7.An FTP application transfers 3 GB of data over a network with an effective throughput of 120 Mbps. The Application Layer divides the data into requests of 3 MB each. Calculate the total number of requests generated and estimate the total transmission time in seconds. Discuss how the Application Layer provides reliable services to users.

8.A packet experiences the following processing delays at different OSI layers: Application Layer = 4 ms, Transport Layer = 3 ms, Network Layer = 5 ms, Data Link Layer = 2 ms, and Physical Layer = 1 ms. The propagation delay is 18 ms, and the transmission delay is 12 ms. Calculate the total end-to-end delay experienced by the packet and explain how each layer contributes to the communication process.

9.A network transfers 80 MB of useful data using frames of 1600 bytes, where each frame contains 80 bytes of protocol overhead added by different OSI layers. Calculate the total number of frames required, the total overhead transmitted, and the transmission efficiency as a percentage. Explain how protocol overhead affects network performance.

10.A hospital transfers a patient's medical record of 240 MB through an IoT-enabled healthcare network. The Presentation Layer compresses the data by 30%, the Transport Layer divides the compressed data into 1400-byte segments, and the Data Link Layer adds 60 bytes of overhead to each frame. Calculate the compressed file size, the total number of segments transmitted, the total protocol overhead introduced, and the final amount of data transmitted across the Physical Layer. Explain how multiple OSI layers work together to

ensure secure and reliable communication.

# RUBRICS FOR CALCULATION

| Criteria | Weightag e | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understandin g of Concepts | 25% | Demonstrates a thorough, accurate understandin g of concepts | Good understanding with minor gaps | Basic understanding with some misconception s | Poor understanding; major misconception s |
| Explanation | 25% | Detailed, well-explaine d answers with relevant examples | Clear explanation with adequate details | Limited explanation; lacks depth | Poor or unclear explanation |
| Application | 20% | Effectively applies concepts with strong analytical ability | Applies concepts with minor errors | Limited application with weak analysis | No application or incorrect analysis |
| Organization | 15% | Well-structure d answers with logical flow and coherence | Organized with minor issues in flow | Basic structure, lacks clarity | Poor organization, incoherent answers |
| Accuracy | 10% | Completely accurate and covers all required points | Mostly accurate with minor omissions | Partially correct with missing points | Incorrect answers with major gaps |
| Clarity | 5% | Clear, neat, professional presentation | Understandabl e with minor issues | Limited clarity, readability issues | Poorly written, difficult to understand |

1. Design File Transfer

Given

- File size = 150 MB = 150,000,000 bytes

- Bandwidth = 50 Mbps

- Segment size = 1500 bytes

- Data Link overhead = 40 bytes/frame

Calculation

Number of Segments

= 150,000,000 / 1500

= 100,000 segments

Number of Frames

= 100,000 frames (1 segment = 1 frame)

Total Overhead

= 100,000 × 40

= 4,000,000 bytes (4 MB)

Total Data Sent

= 150 MB + 4 MB

= 154 MB

Transmission Time

154 MB = 154 × 8 = 1232 Mb

Time = 1232 / 50

= 24.64 seconds

Answer

- Segments = 100,000

- Frames = 100,000

- Overhead = 4 MB

- Transmission Time = 24.64 seconds

Explanation

- Transport Layer divides the file into segments and ensures reliable delivery using acknowledgements and retransmissions.

- Data Link Layer converts segments into frames, detects errors using CRC, and controls communication between adjacent devices.

2. Sliding Window Protocol

Given

- Window size = 6 frames

- Total frames = 48

- Frame size = 1024 bytes

Calculation

Transmission Windows

48 ÷ 6

= 8 windows

Retransmissions

1 frame lost in each window

= 8 retransmissions

Answer

- Windows required = 8

- Retransmissions = 8 frames

Explanation

Flow control prevents the sender from transmitting data faster than the receiver can process, reducing congestion and improving communication efficiency.

3. IP Fragmentation

Given

- Packet size = 12,000 bytes

- MTU = 1500 bytes

- IP header = 20 bytes

Calculation

Maximum data per fragment

= 1500 − 20

= 1480 bytes

Fragments

= Ceiling(12000 / 1480)

= 9 fragments

Header overhead

= 9 × 20

= 180 bytes

Answer

- Fragments = 9

- Header Overhead = 180 bytes

Explanation

The Network Layer fragments packets when they exceed the MTU. Each fragment carries its own header. The destination reassembles all fragments using identification, offset, and flag fields.

4. Sliding Window Protocol

Answer

Since Question 4 is identical to Question 2:

- Windows = 8

- Retransmissions = 8 frames

Explanation

Flow control allows the sender to transmit only the amount of data the receiver can accept, improving reliability and avoiding buffer overflow.

5. Session Layer Synchronization

Given

- File = 600 MB

- Checkpoint every 60 MB

- Failure after 390 MB

Calculation

Checkpoints before failure

390 ÷ 60 = 6 remainder 30

Completed checkpoints = 6

Last checkpoint = 360 MB

Retransmission

390 − 360 = 30 MB already sent after checkpoint.

Remaining data to resend

= 30 MB

Answer

- Checkpoints created = 6

- Data retransmitted = 30 MB

Explanation

Synchronization checkpoints allow communication to resume from the last checkpoint instead of restarting the entire transmission.

6. Presentation Layer Compression

Given

- Original file = 900 MB

- Compression = 40%

- Encryption overhead = 5%

Calculation

Compressed size

= 900 × 60%

= 540 MB

Encrypted size

= 540 × 1.05

= 567 MB

Reduction

900 − 567 = 333 MB

Percentage reduction

= (333 / 900) ×100

= 37%

Answer

- Compressed size = 540 MB

- Encrypted size = 567 MB

- Total reduction = 37%

Explanation

The Presentation Layer compresses data to reduce transmission size and encrypts it to provide confidentiality and secure communication.

7. FTP Data Transfer

Given

- Data = 3 GB = 3000 MB

- Throughput = 120 Mbps

- Request size = 3 MB

Calculation

Requests

3000 ÷ 3

= 1000 requests

Transmission time

3000 MB ×8

= 24,000 Mb

Time

24,000 ÷120

= 200 seconds

Answer

- Requests = 1000

- Transmission Time = 200 seconds

Explanation

The Application Layer provides services such as FTP for file transfer, user authentication, error reporting, and reliable data exchange.

8. End-to-End Delay

Given

- Application = 4 ms

- Transport = 3 ms

- Network = 5 ms

- Data Link = 2 ms

- Physical = 1 ms

- Propagation = 18 ms

- Transmission = 12 ms

Calculation

Total

= 4+3+5+2+1+18+12

= 45 ms

Answer

Total End-to-End Delay = 45 ms

Explanation

Each OSI layer performs specific functions such as application services, segmentation, routing, framing, and signal transmission, contributing to the total communication delay.

9. Protocol Overhead

Given

- Useful data = 80 MB = 80,000,000 bytes

- Frame size = 1600 bytes

- Overhead = 80 bytes

Calculation

Useful data per frame

1600−80

= 1520 bytes

Frames

80,000,000 ÷1520

≈ 52,632 frames

Total overhead

52,632 ×80

= 4,210,560 bytes

Transmission efficiency

=(80,000,000 /84,210,560)×100

≈ 95%

Answer

- Frames = 52,632

- Total overhead = 4,210,560 bytes

- Efficiency = 95%

Explanation

Protocol overhead provides addressing, error detection, and control information but slightly reduces the efficiency of useful data transmission.

10. Hospital Medical Record Transfer

Given

- Original file = 240 MB

- Compression = 30%

- Segment size = 1400 bytes

- Frame overhead = 60 bytes

Calculation

Compressed size

240 ×70%

= 168 MB

Compressed bytes

168 MB

= 168,000,000 bytes

Segments

168,000,000 ÷1400

= 120,000 segments

Protocol overhead

120,000 ×60

= 7,200,000 bytes

Final transmitted data

168,000,000 +7,200,000

= 175,200,000 bytes

= 175.2 MB

Answer

- Compressed file = 168 MB

- Segments = 120,000

- Protocol overhead = 7.2 MB

- Final transmitted data = 175.2 MB

Explanation

The Presentation Layer compresses data to reduce its size, the Transport Layer divides it into segments for reliable delivery, the Data Link Layer adds framing and error detection information, and the Physical Layer transmits the final data over the communication medium. Together, these layers ensure secure, reliable, and efficient communication.